APPENDIX  G

Program PLFRAME for the Analysis of Rigid-jointed Plane Frames

This program PLFRAME.FOR written in Fortran is for the analysis of plane frames by direct stiffness method. The input data is given in the file 2dfin.dat and the output is stored in the file 2dfout.dat. The input data must be given in the following form.

1.  HEADING

2.  NJ,NE

NJ- NUMBER OF JOINTS

NE- NUMBER OF ELEMENTS

3.  NODAL DATA

NODE NO, KODE, X,Y,FX,FY,FZ

KODE- U V ROTN. IT IS A THREE DIGIT NUMBER AND IF ANY DIGIT IS 1 IT

SHOWS PARTICULAR DISPLACEMENT IS CONSTRAINED

FOR EXAMPLE

111- FIXED

110- HINGED

010- ROLLER IN HORIZONTAL DIRECTION (Y-DISP IS GIVEN)

100- ROLLER IN VERTICAL DIRECTION (X-DISP IS GIVEN)

4.  ELEMENT DATA

ELNO, I, J, AREA,MI

5.  FIXED END STRESS RESULTANTS (MUST BE GIVEN FOR LOADED MEMBER)

THE DATA MUST BE TERMINATED WITH ZERO

PI,VI,MI,PJ,VJ,MJ

PI- AXIAL FORCE AT ITH JOINT + ALONG THE AXIS

VI- SHEAR FORCE AT ITH JOINT + IN THE POSITIVE DIRECTION

MI- MOMENT AT I ABOUT Z AXIS + IN THE ANTICLOCKWISE DIRECTION

PJ,VJ,MJ IS FOR THE JTH JOINT

OUTPUT FILE CONTAINS DISPLACEMENTS AT VARIOUS NODES AND STRESS RESULTANTS OF VARIOUS MEMBERS.

The input data given here is for the example shown in Fig. G.1.

PROGRAM PLFRAME.FOR

$debug

c	PROGRAM PLFRAME

c	Program for the Static Analysis of Simple Framed Structures

common x(100),y(100),u(100),v(100),r(100),area(200),ri(200),

*ymod(200), b(300),a(300,90),hed(20),

* s(6,6),fef(6,200),fq(6)

common ne,nj,neq,mb,kode(100),nodi(200),nodj(200),

* lm(6),in,io,maxe

maxe=100

open( 1,file= '2dfin.dat' ,status='old')

open( 2,file= '2dfout.dat',status='new')

open( 3,file= '2dfplt.dat',status='new')

c

c

call input

c

c

call stiff

c

call displ

c

call stress

c

end

subroutine input

c

common x(100),y(100),u(100),v(100),r(100),area(200),ri(200),

*ymod(200), b(300),a(300,90),hed(20),

* s(6,6),fef(6,200),fq(6)

common ne,nj,neq,mb,kode(100),nodi(200),nodj(200),

* lm(6),in,io,maxe

in=1

io=2

njmax=100

nemax=200

mbmax=90

c

read(in,1000) hed,nj,ne

if(nj.gt.njmax.or.ne.gt.nemax) go to 999

write( io,2000) hed,nj,ne

write(3,'(20a4/2i5)') hed,nj,ne

c

write( io,2001)

do 100 m=1,nj

read(in,1001)n,kode(n),x(n),y(n),u(n),v(n),r(n)

write(3,'(2i5,2e15.7)') n,kode(n),x(n),y(n)

100  write( io,2002) n,kode(n),x(n),y(n),u(n),v(n),r(n)

c

write( io,2003)

ndif=0

do 200 n=1,ne

read(in,1002) m,nodi(m),nodj(m),area(m),ri(m),ymod(m)

k=iabs(nodi(m)-nodj(m))

if(k.gt.ndif)ndif=k

write(3,'(3i5)')m,nodi(m),nodj(m)

200	 write( io,2004) m,nodi(m),nodj(m),area(m),ri(m),ymod(m)

mb=3*(ndif+1)

neq=3*nj

do 800 i=1,ne

do 800 j=1,6

800  fef(j,i)=0.0

write(io,*)' stress resultants for fixed state'

write(io,5)

5  format(2x,' m',15x,'ti',15x,'vi',15x,'mi',15x,'tj',15x,'vj',

*15x,'mj',/)

600  read(in,300)m

300  format(i5)

if(m .eq. 0) goto 520

read(in,400)(fef(i,m),i=1,6)

write(io,500)m,(fef(i,m),i=1,6)

go to 600

400  format(6f10.0)

500  format(i5,6(e15.7,4x))

c

520  return

c

999  write(io,1999)

stop

c

c	format statements

1000  format(20a4/2i5,f10.0)

1001  format(2i5,5f12.0)

1002  format(3i5,3f12.0)

1100  format(2i5,6f10.0)

2000  format('1',20a4/'  **number of joints     =',i4/

*                '  **number of elements   =',i4/)

2001 format(' echo check of joint data'/' joint kode',6x,'x',llx

*,'y',10x,'rx',10x,'ry',10x,'rm')

2002  format(2i5,5e12.4)

2003  format('0',' echo check of element data'/' memb nodi nodj ',7x

*'area',7x,'inertia',6x,'modulus',2x,'status')

2004  format(3i5,3e15.5)

2200  format(3i5,6f11.4)

1999  format('  *** parameters exceed those permitted by program **')

end

subroutine stiff

c

c

common x(100),y(100),u(100),v(100),r(100),area(200),ri(200),

*ymod(200), b(300),a(300,90),hed(20),

* s(6,6),fef(6,200),fq(6)

common ne,nj,neq,mb,kode(100),nodi(200),nodj(200),

* lm(6),in,io,maxe

dimension t(3,6),cm(3,3),temp(3,6)

data t(1,6),t(2,3),t(3,3),t(3,6),cm(1,3),cm(2,3),cm(3,1),

*cm(3,2)/8*0.0/,t(1,3),t(2,6)/2*1.0/

c

c	initialize stiffness matrix

do 50 j=1,mb

do 50 i=1,neq

50 a(i,j)=0.0

do 100 i=1,nj

ii=3*i

b(ii)=r(i)

b(ii-1)=v(i)

100 b(ii-2)=u(i)

do 500 m=1,ne

c

c	form member stiffness matrix

i=nodi(m)

j=nodj(m)

dx=x(j)-x(i)

dy=y(j)-y(i)

xl=sqrt(dx**2+dy**2)

xli=1./xl

alp=ymod(m)*ri(m)*xli

beta=ymod(m)*area(m)*xli

co=dx*xli

si=dy*xli

cl=co*xli

sl=si*xli

t(1,1)=-sl

t(1,2)=cl

t(1,4)=sl

t(1,5)=-cl

t(2,1)=-sl

t(2,2)=cl

t(2,4)=sl

t(2,5)=-cl

t(3,1)=-co

t(3,2)=-si

t(3,4)=co

t(3,5)=si

cm(3,3)=beta

110  cm(1,1)=4.*alp

cm(1,2)=2.*alp

cm(2,1)=cm(1,2)

cm(2,2)=cm(1,1)

c

200 do 300 l=1,6

do 300 k=1,3

d=0.0

do 250 n=1,3

250   d=d+cm(k,n)*t(n,l)

300  temp(k,l)=d

c

do 400 l=1,6

do 400 k=l,6

d=0.0

do 350 n=1,3

350 d=d+t(n,l)*temp(n,k)

s(l,k)=d

400 s(k,l)=d

425 continue

do 700 ji=1,6

700  fq(ji)=fef(ji,m)

ii=3*i

b(ii-2)=b(ii-2)-fq(1)*co+fq(2)*si

b(ii-1)=b(ii-1)-fq(1)*si-fq(2)*co

b(ii)=b(ii)-fq(3)

jj=3*j

b(jj-2)=b(jj-2)-fq(4)*co+fq(5)*si

b(jj-1)=b(jj-1)-fq(4)*si-fq(5)*co

b(jj)=b(jj)-fq(6)

c	assemble stiffness matrix

lm(3)=3*nodi(m)

lm(2)=lm(3)-1

lm(1)=lm(3)-2

lm(6)=3*nodj(m)

lm(5)=lm(6)-1

lm(4)=lm(6)-2

do 450 i=1,6

ii=lm(i)

do 450 j=1,6

jj=lm(j)-ii+1

if(jj.le.0) goto 450

a(ii,jj)=a(ii,jj)+s(i,j)

450 continue

500 continue

c

do 600 n=1,nj

if(kode(n).eq.0) goto 600

k=kode(n)

ex=0.0

ey=0.0

er=0.0

if((k-100).lt.0) goto 520

ex=1.0e20

k=k-100

520 if (k-10.lt.0) goto 540

ey=1.0e20

k=k-10

540 if (k.eq.0) goto 550

er=1.0e20

550 nn=3*n

a(nn,1)=a(nn,1)+er

a(nn-1,1)=a(nn-1,1)+ey

a(nn-2,1)=a(nn-2,1)+ex

b(nn)=b(nn)+er*r(n)

b(nn-1)=b(nn-1)+ey*v(n)

b(nn-2)=b(nn-2)+ex*u(n)

600 continue

c

return

end

subroutine displ

c

common x(100),y(100),u(100),v(100),r(100),area(200),ri(200),

*ymod(200), b(300),a(300,90),hed(20),

* s(6,6),fef(6,200),fq(6)

common ne,nj,neq,mb,kode(100),nodi(200),nodj(200),

* lm(6),in,io,maxe

c

call band1

c

do 100 n=1,nj

nn=3*n

u(n)=b(nn-2)

v(n)=b(nn-1)

100 r(n)=b(nn)

c

write(io,2000)

write(3,'(i5,3e15.7)')(n,u(n),v(n),r(n),n=1,nj)

write(io,2100) (n,u(n),v(n),r(n),n=1,nj)

2000 format('1',' *** nodal displacements ***'/'     joint',6x,'u',

*14x,'v',10x,'rotation')

2100 format(i10,3e15.6)

c

return

end

subroutine stress

c

common x(100),y(100),u(100),v(100),r(100),area(200),ri(200),

*ymod(200), b(300),a(300,90),hed(20),

* s(6,6),fef(6,200),fq(6)

common ne,nj,neq,mb,kode(100),nodi(200),nodj(200),

* lm(6),in,io,maxe

c

c

do 500 m=1,ne

i=nodi(m)

j=nodj(m)

dx=x(j)-x(i)

dy=y(j)-y(i)

du=u(j)-u(i)

dv=v(j)-v(i)

xli=1./sqrt(dx**2+dy**2)

cost=dx*xli

sint=dy*xli

c1=2.*ymod(m)*ri(m)*xli

dp=area(m)*ymod(m)*xli*(du*cost+dv*sint)

rot=3.*(dv*cost-du*sint)*xli

dmi=c1*(2.*r(i)+r(j)-rot)

dmj=c1*(2.*r(j)+r(i)-rot)

dv=(dmi+dmj)*xli

c

c	modify end forces for member code

fef(1,m)=fef(1,m)-dp

fef(2,m)=fef(2,m)+dv

fef(3,m)=fef(3,m)+dmi

fef(4,m)=fef(4,m)+dp

fef(5,m)=fef(5,m)-dv

fef(6,m)=fef(6,m)+dmj

500 continue

c	output member forces

write(io,1000)

write(io,1010)(m,(fef(i,m),i=1,6),m=1,ne)

c

return

c

1000 format('1','***final member end forces***'/

*' mem',5x,'ti',9x,'vi',9x,'mi',9x,'tj',9x,'vj',9x,'mj')

1010 format(i4,6f11.3)

c

end

subroutine band1

c

c	this subroutine solves for displacements using gaussian elimination

c	technique for symmetric banded matrices stored in core

c

c	triangularize and reduce right hand side

common x(100),y(100),u(100),v(100),r(100),area(200),ri(200),

*ymod(200), b(300),a(300,90),hed(20),

* s(6,6),fef(6,200),fq(6)

common ne,nj,nn,mm,kode(100),nodi(200),nodj(200),

* lm(6),in,io,maxe

nl=nn-mm+1

nm=nn-1

mr=mm

c

do 100 n=1,nm

if(a(n,1).le.0.) goto 700

bn=b(n)

b(n)=bn/a(n,1)

if(n.gt.nl) mr=nn-n+1

c

do 100 l=2,mr

if(a(n,l).eq.0.) goto 100

c=(a(n,l)/a(n,1))

i=n+l-1

j=0

do 50 k=l,mr

j=j+1

50 a(i,j)=a(i,j)-c*a(n,k)

b(i)=b(i)-c*bn

a(n,l)=c

100 continue

c

c	back substitute

c

i=nn

c

b(nn)=b(nn)/a(nn,1)

do 600 n=1,nm

i=i-1

if (n.lt.mm)mr=n+1

do 600 j=2,mr

k=i+j-1

600 b(i)=b(i)-a(i,j)*b(k)

return

700 write( 2,2000)n

stop

2000 format(1ho,' zero or negative element on main diagonal of tr

*iangularized matrix for equation ',i5)

c

end

c

Input File 2dfin.dat (see Fig. G.1)

PLANE FRAME

4,3

1,111,0,0

2,0,0,8,-10

3,0,6,8

4,110,6,0

1,1,2,1000,10,200000

2,2,3,1000,10,200000

3,3,4,1000,10,200000

1

0,24,32,0,24,-32

2

0,12,18,0,12,-18

0

Output File 2dfout.dat

1PLANE FRAME

**number of joints     =   4

**number of elements   =   3

echo check of joint data

joint     kode             x                  y                       rx                 ry                  rm

1        111      .0000E+00     .0000E+00     .0000E+00     .0000E+00     .0000E+00

2          0        .0000E+00     .8000E+01    -.1000E+02     .0000E+00     .0000E+00

3          0        .6000E+01     .8000E+01     .0000E+00     .0000E+00     .0000E+00

4       110       .6000E+01     .0000E+00     .0000E+00     .0000E+00     .0000E+00

0 echo check of element data

memb   nodi     nodj              area                     inertia         modulus  status

1          1          2           .10000E+04        .10000E+02        .20000E+06

2          2          3           .10000E+04        .10000E+02        .20000E+06

3          3          4           .10000E+04        .10000E+02        .20000E+06

stress resultants for fixed state

m               ti                           vi                       mi                          tj                            vj                          mj

1   .0000000E+00     .2400000E+02      .3200000E+02      .0000000E+00      .2400000E+02     -.3200000E+02

2   .0000000E+00     .1200000E+02      .1800000E+02      .0000000E+00      .1200000E+02     -.1800000E+02

1 *** nodal displacements ***

joint               u                     v                 rotation

1    .344423E-18   -.638081E-19   -.782911E-18

2    .295121E-03   -.255232E-06   -.180880E-04

3    .295014E-03   -.704767E-06    .108061E-05

4    .355850E-19   -.176192E-18   -.558554E-04

1***final member end forces***

mem      ti             vi            mi               tj              vj              mj

1       6.381     34.442     78.291       -6.381      13.558         5.247

2       3.558       6.381      -5.247       -3.558      17.619     -28.468

3     17.619       3.559      28.468    -17.619       -3.559          .000