APPENDIX H

Program GRID for the Analysis of Grids

The program GRID.FOR written in Fortran is for the analysis of grids by the direct stiffness method.  The input file is gridin.dat and the output file gridout.dat. The input data must be given in the following form.

INPUT DATA

1. HEADING

2. NJ,NE

NJ- NUMBER OF JOINTS

NE- NUMBER OF ELEMENTS

3. NODAL DATA

N,KODE,X,Y,MX,MY,RZ

N- NODE NO

X,Y- COORDINATES OF JOINT N

MX,MY,RZ - FORCES - (MOMENT ABOUT X AND Y AXES RESPLY) VERTICAL FORCE

4. ELEMENT DATA

ELNO, I,J, TOR INERTIA, MI, E,G

ELNO- ELEMENT NUMBER

I- ITH NODE OF THE MEMBER

J- JTH NODE OF THE MEMBER

TOR INERTIA- TORSIONAL MOMENT OF INERTIA

MI- MOMENT OF INERTIA

E=YOUNGS MODULUS

G=RIGIDITY MODULUS

5. FIXED END MOMENTS

GIVE DATA FOR ELEMENTS HAVING LOADS

THE DATA MUST BE TERMINATED WITH ZERO

TI,MI,VI,TJ,MJ,VJ

TI- FIXED END TORSIONAL MOMENT AT I

MI- FIXED END MOMENT AT I

VI- FIXED END SHEAR AT I

TJ,MJ,VJ IS FOR THE END J

The data given is for the example shown in Fig. H.1.

PROGRAM GRID.FOR

c	PROGRAM GRID

c	program for the static analysis of simple framed structures

common x(100),y(100),u(100),v(100),r(100),area(200),ri(200),

*ymod(200), b(300),a(300,90),hed(20),

* s(6,6),fef(6,200),fq(6)

common ne,nj,neq,mb,kode(100),nodi(200),nodj(200),

* lm(6),in,io,maxe,gmod(200)

maxe=100

open( 1,file= 'gridin.dat',status='old')

open( 2,file= 'gridout.dat',status='new')

c

c

call input

c

c

call stiff

c

call displ

c

call stress

c

end

subroutine input

c

common x(100),y(100),u(100),v(100),r(100),area(200),ri(200),

*ymod(200), b(300),a(300,90),hed(20),

* s(6,6),fef(6,200),fq(6)

common ne,nj,neq,mb,kode(100),nodi(200),nodj(200),

* lm(6),in,io,maxe,gmod(200)

in=1

io=2

njmax=100

nemax=200

mbmax=90

c

read(in,1000) hed,nj,ne

if(nj.gt.100.or.ne.gt.200)GO TO 999

write( io,2000) hed,nj,ne

c

write( io,2001)

do 100 m=1,nj

read(in,1001)n,kode(n),x(n),y(n),u(n),v(n),r(n)

100  write( io,2002) n,kode(n),x(n),y(n),u(n),v(n),r(n)

c

write( io,2003)

ndif=0

do 200 n=1,ne

read(in,1002) m,nodi(m),nodj(m),area(m),ri(m),ymod(m),gmod(m)

k=iabs(nodi(m)-nodj(m))

if(k.gt.ndif)ndif=k

200	 write( io,2004) m,nodi(m),nodj(m),area(m),ri(m),ymod(m), gmod (m)

mb=3*(ndif+1)

neq=3*nj

do 800 i=1,ne

do 800 j=1,6

800  fef(j,i)=0.0

write(io,*)' stress resultants for fixed state'

write(io,5)

5  format(2x,' m',15x,'ti',15x,'mi',15x,'vi',15x,'tj',15x,'mj',

*15x,'vj',/)

600  read(in,300)m

300  format(i5)

if(m .eq. 0) goto 520

read(in,400)(fef(i,m),i=1,6)

write(io,500)m,(fef(i,m),i=1,6)

goto 600

400  format(6f10.0)

500  format(i5,6(e15.7,4x))

c

520  return

c

999  write(io,1999)

stop

c

c	format statements

1000  format(20a4/2i5)

1001  format(2i5,5f10.0)

1002  format(3i5,4f10.0)

1100  format(2i5,6f10.0)

2000  format('1',20a4/'  **number of joints     =',i4/

*                '  **number of elements   =',i4/)

2001 format(' echo check of joint data'/' joint kode',6x,'x',llx

*,'y',10x,'mx',10x,'my',10x,'rz')

2002  format(2i5,5e12.4)

2003  format('0',' echo check of element data'/' memb nodi nodj ',7x

*'tor iner',7x,'m.i',6x,'  e    ',2x,'  g   ')

2004  format(3i5,4e15.5)

2200  format(3i5,6f11.4)

1999  format('  *** parameters exceed those permitted by program **')

end

subroutine stiff

c

c

common x(100),y(100),u(100),v(100),r(100),area(200),ri(200),

*ymod(200), b(300),a(300,90),hed(20),

* s(6,6),fef(6,200),fq(6)

common ne,nj,neq,mb,kode(100),nodi(200),nodj(200),

* lm(6),in,io,maxe,gmod(200)

dimension t(3,6),cm(3,3),temp(3,6)

data t(1,4),t(1,5),t(3,3),t(3,6),cm(1,3),cm(2,3),cm(3,1),

*cm(3,2)/8*0.0/

c

c	initialize stiffness matrix

do 50 j=1,mb

do 50 i=1,neq

50 a(i,j)=0.0

do 100 i=1,nj

ii=3*i

b(ii)=r(i)

b(ii-1)=v(i)

100 b(ii-2)=u(i)

do 500 m=1,ne

c

c	form member stiffness matrix

i=nodi(m)

j=nodj(m)

dx=x(j)-x(i)

dy=y(j)-y(i)

xl=sqrt(dx**2+dy**2)

xli=1./xl

alp=ymod(m)*ri(m)*xli

beta=gmod(m)*area(m)*xli

co=dx*xli

si=dy*xli

cl=co*xli

sl=si*xli

t(1,1)=-si

t(1,2)=co

t(1,3)=-xli

t(1,6)=xli

t(2,1)=0.0

t(2,2)=0.0

t(2,3)=-xli

t(2,4)=-si

t(2,5)=co

t(2,6)=xli

t(3,1)=-co

t(3,2)=-si

t(3,4)=co

t(3,5)=si

cm(3,3)=beta

110  cm(1,1)=4.*alp

cm(1,2)=2.*alp

cm(2,1)=cm(1,2)

cm(2,2)=cm(1,1)

c

200 do 300 l=1,6

do 300 k=1,3

d=0.0

do 250 n=1,3

250   d=d+cm(k,n)*t(n,l)

300  temp(k,l)=d

c

do 400 l=1,6

do 400 k=l,6

d=0.0

do 350 n=1,3

350 d=d+t(n,l)*temp(n,k)

s(l,k)=d

400 s(k,l)=d

425 continue

do 700 ji=1,6

700  fq(ji)=fef(ji,m)

ii=3*i

b(ii-2)=b(ii-2)-fq(1)*co+fq(2)*si

b(ii-1)=b(ii-1)-fq(1)*si-fq(2)*co

b(ii)=b(ii)-fq(3)

jj=3*j

b(jj-2)=b(jj-2)-fq(4)*co+fq(5)*si

b(jj-1)=b(jj-1)-fq(4)*si-fq(5)*co

b(jj)=b(jj)-fq(6)

c	assemble stiffness matrix

lm(3)=3*nodi(m)

lm(2)=lm(3)-1

lm(1)=lm(3)-2

lm(6)=3*nodj(m)

lm(5)=lm(6)-1

lm(4)=lm(6)-2

do 450 i=1,6

ii=lm(i)

do 450 j=1,6

jj=lm(j)-ii+1

if(jj.le.0) goto 450

a(ii,jj)=a(ii,jj)+s(i,j)

450 continue

500 continue

c

do 600 n=1,nj

if(kode(n).eq.0) goto 600

k=kode(n)

ex=0.0

ey=0.0

er=0.0

if((k-100).lt.0) goto 520

ex=1.0e20

k=k-100

520 if (k-10.lt.0) goto 540

ey=1.0e20

k=k-10

540 if (k.eq.0) goto 550

er=1.0e20

550 nn=3*n

a(nn,1)=a(nn,1)+er

a(nn-1,1)=a(nn-1,1)+ey

a(nn-2,1)=a(nn-2,1)+ex

b(nn)=b(nn)+er*r(n)

b(nn-1)=b(nn-1)+ey*v(n)

b(nn-2)=b(nn-2)+ex*u(n)

600 continue

c

return

end

subroutine displ

c

common x(100),y(100),u(100),v(100),r(100),area(200),ri(200),

*ymod(200), b(300),a(300,90),hed(20),

* s(6,6),fef(6,200),fq(6)

common ne,nj,neq,mb,kode(100),nodi(200),nodj(200),

* lm(6),in,io,maxe,gmod(200)

c

call band1

c

do 100 n=1,nj

nn=3*n

u(n)=b(nn-2)

v(n)=b(nn-1)

100 r(n)=b(nn)

c

write(io,2000)

write(io,2100) (n,u(n),v(n),r(n),n=1,nj)

2000 format('1',' *** nodal displacements ***'/'     joint',6x,'rot-x',

*14x,'rot-y',10x,'z-disp')

2100 format(i10,3e15.6)

c

return

end

subroutine stress

c

common x(100),y(100),u(100),v(100),r(100),area(200),ri(200),

*ymod(200), b(300),a(300,90),hed(20),

* s(6,6),fef(6,200),fq(6)

common ne,nj,neq,mb,kode(100),nodi(200),nodj(200),

* lm(6),in,io,maxe,gmod(200)

c

c

do 500 m=1,ne

i=nodi(m)

j=nodj(m)

dx=x(j)-x(i)

dy=y(j)-y(i)

du=u(j)-u(i)

dv=v(j)-v(i)

xli=1./sqrt(dx**2+dy**2)

cost=dx*xli

sint=dy*xli

c1=2.*ymod(m)*ri(m)*xli

dp=area(m)*gmod(m)*xli*(du*cost+dv*sint)

rot=3.*(r(j)-r(i))*xli

dmi=c1*((2.0*v(i)+v(j))*cost-(2.0*u(i)+u(j))*sint+rot)

dmj=c1*((2.0*v(j)+v(i))*cost-(2.0*u(j)+u(i))*sint+rot)

dv=(dmi+dmj)*xli

c

c	modify end forces for member code

fef(1,m)=fef(1,m)-dp

fef(2,m)=fef(2,m)+dmi

fef(3,m)=fef(3,m)-dv

fef(4,m)=fef(4,m)+dp

fef(5,m)=fef(5,m)+dmj

fef(6,m)=fef(6,m)+dv

500 continue

c	output member forces

write(io,1000)

write(io,1010)(m,(fef(i,m),i=1,6),m=1,ne)

c

return

c

1000 format('1','***final member end forces***'/

*' mem',5x,'ti',9x,'mi',9x,'vi',9x,'tj',9x,'mj',9x,'vj')

1010 format(i4,6f11.3)

c

end

subroutine band1

c

c	this subroutine solves for displacements using gaussian elimination

c	technique for symmetric banded matrices stored in core

c

c	triangularize and reduce right hand side

common x(100),y(100),u(100),v(100),r(100),area(200),ri(200),

*ymod(200), b(300),a(300,90),hed(20),

* s(6,6),fef(6,200),fq(6)

common ne,nj,nn,mm,kode(100),nodi(200),nodj(200),

* lm(6),in,io,maxe,gmod(200)

nl=nn-mm+1

nm=nn-1

mr=mm

c

do 100 n=1,nm

if(a(n,1).le.0.) goto 700

bn=b(n)

b(n)=bn/a(n,1)

if(n.gt.nl) mr=nn-n+1

c

do 100 l=2,mr

if(a(n,l).eq.0.) goto 100

c=(a(n,l)/a(n,1))

i=n+l-1

j=0

do 50 k=l,mr

j=j+1

50 a(i,j)=a(i,j)-c*a(n,k)

b(i)=b(i)-c*bn

a(n,l)=c

100 continue

c

c	back substitute

c

i=nn

c

b(nn)=b(nn)/a(nn,1)

do 600 n=1,nm

i=i-1

if (n.lt.mm)mr=n+1

do 600 j=2,mr

k=i+j-1

600 b(i)=b(i)-a(i,j)*b(k)

return

700 write( 2,2000)n

stop

2000 format(1ho,' zero or negative element on main diagonal of tr

*iangularized matrix for equation ',i5)

c

end

c

Input File gridin.dat (see Fig. H.1)

grid problem

4,3

1,111,0,0,0,0,0

2,0,10,0,0,0,0

3,0,10,10,0,0,0

4,111,0,10,0,0,0

1,1,2,500,1000,30000.0,12000.0

2,2,3,500,1000,30000.0,12000.0

3,3,4,500,1000,30000.0,12000.0

2

0,41.67,25,0,-41.67,25

0

Output File gridout.dat

1  grid problem

**number of joints     =   4

**number of elements   =   3

echo check of joint data

joint   kode      x                     y                  mx                  my              rz

1     111   .0000E+00   .0000E+00   .0000E+00   .0000E+00   .0000E+00

2      0      .1000E+02   .0000E+00   .0000E+00   .0000E+00   .0000E+00

3      0      .1000E+02   .1000E+02   .0000E+00   .0000E+00   .0000E+00

4    111    .0000E+00   .1000E+02   .0000E+00   .0000E+00   .0000E+00

0 echo check of element data

memb   nodi    nodj        tor. iner                m.i                    e	                         g

1          1         2       .50000E+03     .10000E+04     .30000E+05       .12000E+05

2          2         3       .50000E+03     .10000E+04     .30000E+05       .12000E+05

3          3         4       .50000E+03     .10000E+04     .30000E+05       .12000E+05

stress resultants for fixed state

m               ti               mi               vi               tj               mj               vj

2   .0000000E+00       .4167000E+02       .2500000E+02       .0000000E+00      -.4167000E+02       .2500000E+02

1 *** nodal displacements ***

joint          rot-x                 rot-y               z-disp

1    .378819E-19    .250000E-17   -.250000E-18

2    .631366E-05    .416667E-04   -.277778E-03

3   -.631362E-05    .416666E-04   -.277778E-03

4   -.378817E-19    .250000E-17   -.250000E-18

1***final member end forces***

mem         ti              mi               vi            tj              mj               vj

1        -3.788     -250.000      25.000      3.788            .000      -25.000

2           .000          3.788       25.000        .000        -3.788        25.000

3         3.788           .000       -25.000     -3.788    250.000         25.000