APPENDIX I

Program 3DTRUSS for the Analysis of Pin-jointed Space Trusses

The program 3dtruss written in Fortran solves any three-dimensional pin-jointed truss. The element stiffness is assembled in band form and the solution of equations is carried out using Gaussian elimination method for band matrix. Copy your input data file 3dtin.dat and the output file is t3dout1.dat. The input data must be given in the following form.

1.  HEADING

2.  NJ,NE

NJ- NO OF JOINTS

NE- NO OF ELEMENTS

3.  NODAL INFORMATION

NODE NO, KODE, X, Y,Z,FX,FY,FZ

KODE FOR U,V,W AND IT IS THREE DIGIT NUMBER AND IF 1 IN ANY DIGIT IT SHOWS

THAT PARTICULAR DISPLACEMENT IS SPECIFIED OTHERWISE IT IS ZERO.

4.  ELEMENT INFORMATION

ELNO, NODI, NODJ, AREA, YOUNGS MODULUS

SHOULD BE TERMINATED WITH 0,0,0

The output contains the displacements of various nodes and the forces in various members of the truss. The data given in this example is for the truss shown in Fig. I.1.

PROGRAM 3DTRUSS.FOR

$large

$storage:2

$nofloatcalls

C      THREE DIMENSIONAL TRUSS PROGRAM

common a(200,25),s(6,6),sgw(3,3),area(240),ymod(240),b(240),

*u(63),v(63),w(63),x(63),y(63),z(63),pgw(30),xgw(30),ygw(30),

*  zgw(30),hed(20),t(6),cosa(3),

* kode(63),nodi(240),nodj(240),nodgw(30),lm(6),

* agw,egw,rho,iflag,maxb,maxe,mb,neq,ne,ngw,nj

maxb=25

maxe=200

open(1,file='t3dout1.dat',status='new')

open(2,file='t3dout2.dat',status='new')

open(3,file='plot1.dat',status='new')

open(4,file='plot2.dat',status='new')

do 200 iflag=1,2

open(5,file='3dtin.dat',status='old')

call input

call stiff

call displ

call stress

close(iflag,status='keep')

200    close(5,status='keep')

stop

end

subroutine input

common a(200,25),s(6,6),sgw(3,3),area(240),ymod(240),b(240),

*u(63),v(63),w(63),x(63),y(63),z(63),pgw(30),xgw(30),ygw(30),

*  zgw(30),hed(20),t(6),cosa(3),

* kode(63),nodi(240),nodj(240),nodgw(30),lm(6),

* agw,egw,rho,iflag,maxb,maxe,mb,neq,ne,ngw,nj

read(5,1000)hed,nj,ne,rho

write(iflag,2000)hed,nj,ne,rho

write(3,'(20a4/2i5)')hed,nj,ne

write(iflag,2001)

do 100 n=1,nj

read(5,1001)m,kode(m),x(m),y(m),z(m),u(m),v(m),w(m)

write(3,'(i5,3e15.6)')m,x(m),y(m),z(m)

100    write(iflag,2002)m,kode(m),x(m),y(m),z(m),u(m),v(m),w(m)

ndif=0

write(iflag,2003)

do 200 n=1,ne

read(5,1002)m,nodi(m),nodj(m),area(m),ymod(m)

k=iabs(nodi(m)-nodj(m))

write(3,'(3i5)')m,nodi(m),nodj(m)

if(k.gt.ndif) ndif=k

200    write(iflag,2004)m,nodi(m),nodj(m),area(m),ymod(m)

mb=3*(ndif+1)

if(mb.gt.50) goto 999

read(5,'(i5,2f10.0)')ngw,egw,agw

write(3,'(i5)')ngw

do 300 i=1,ngw

read(5,'(i5,3f10.0)')nodgw(i),xgw(i),ygw(i),zgw(i)

write(iflag,'(i5,3f10.0)')ngw,egw,agw

300     write(3,'(i5,3e15.6)')nodgw(i),xgw(i),ygw(i),zgw(i)

return

999     write(iflag,2005)mb,maxb

stop

1000   format(20a4/2i5,f10.0)

1001   format(2i5,6f10.0)

1002   format(3i5,2f10.0)

2000   format(/////20a4/' Number of joints        = ',i4/

1  ' Number of elements      = ',i4/

2  ' Unit weight of material = ',f10.0)

2001   format(////' Joint no.',6x,'Code',14x,'X',14x,'Y',14x,'Z',

*   13x,'RX',13x,'RY',13x,'RZ')

2002  format(2i10,6e15.6)

2003  format(////'   Element',9x,'I',9x,'J',14x,'A',14x,'E')

2004  format(/3i10,2e15.6)

2005 format(' *** Band width of ',i4,' exceeds allowable of ',i4)

end

subroutine stiff

common a(200,25),s(6,6),sgw(3,3),area(240),ymod(240),b(240),

*u(63),v(63),w(63),x(63),y(63),z(63),pgw(30),xgw(30),ygw(30),

*  zgw(30),hed(20),t(6),cosa(3),

* kode(63),nodi(240),nodj(240),nodgw(30),lm(6),

* agw,egw,rho,iflag,maxb,maxe,mb,neq,ne,ngw,nj

neq=3*nj

if(neq.gt.maxe) goto 999

do 50 i=1,neq

b(i)=0.0

do 50 j=1,mb

50     a(i,j)=0.0

do 500 n=1,ne

i=nodi(n)

j=nodj(n)

dx=x(j)-x(i)

dy=y(j)-y(i)

dz=z(j)-z(i)

xl=sqrt(dx**2+dy**2+dz**2)

cosa(1)=dx/xl

cosa(2)=dy/xl

cosa(3)=dz/xl

comm=area(n)*ymod(n)/xl

do 100 i=1,3

t(i)=-cosa(i)

100    t(i+3)=cosa(i)

do 200 i=1,6

c=t(i)*comm

do 200 j=1,6

s(i,j)=c*t(j)

200    s(j,i)=s(i,j)

lm(3)=3*nodi(n)

lm(2)=lm(3)-1

lm(1)=lm(2)-1

lm(6)=3*nodj(n)

lm(5)=lm(6)-1

lm(4)=lm(5)-1

do 400 i=1,6

ii=lm(i)

do 400 j=1,6

jj=lm(j)-ii+1

if(jj.le.0) goto 400

a(ii,jj)=a(ii,jj)+s(i,j)

400    continue

500    continue

do 600 i=1,ngw

e=egw

if(iflag.eq.2.and.pgw(i).lt.0)e=0.0

dx=xgw(i)-x(nodgw(i))

dy=ygw(i)-y(nodgw(i))

dx=zgw(i)-z(nodgw(i))

xl=sqrt(dx**2+dy**2+dz**2)

sgw(1,1)=e*agw/(xl**3)*(dx*dx)

sgw(1,2)=e*agw/(xl**3)*(dx*dy)

sgw(1,3)=e*agw/(xl**3)*(dx*dz)

sgw(2,1)=sgw(2,1)

sgw(2,2)=e*agw/(xl**3)*(dy*dy)

sgw(2,3)=sgw(3,2)

sgw(3,1)=sgw(1,3)

sgw(3,2)=sgw(2,3)

sgw(3,3)=e*agw/(xl**3)*(dz*dz)

lm(3)=nodgw(i)*3

lm(2)=lm(3)-1

lm(1)=lm(2)-1

do 700 j=1,3

jj=lm(j)

do 700 k=1,3

kk=lm(k)-jj+1

if(kk.lt.0) goto 700

a(jj,kk)=a(jj,kk)+sgw(j,k)

700    continue

600    continue

return

999    write(iflag,1000)neq,maxe

1000 format(' *** Number of equations is ',i4,' which exceeds '

* ' allowable number of ',i4)

stop

end

subroutine displ

common a(200,25),s(6,6),sgw(3,3),area(240),ymod(240),b(240),

*u(63),v(63),w(63),x(63),y(63),z(63),pgw(30),xgw(30),ygw(30),

*  zgw(30),hed(20),t(6),cosa(3),

* kode(63),nodi(240),nodj(240),nodgw(30),lm(6),

* agw,egw,rho,iflag,maxb,maxe,mb,neq,ne,ngw,nj

do 50 k=1,ne

i=nodi(k)

j=nodj(k)

dx=x(j)-x(i)

dy=y(j)-y(i)

dz=z(j)-z(i)

xl=sqrt(dx**2+dy**2+dz**2)

w(i)=w(i)-xl*area(k)*rho/2.0

50     w(j)=w(j)-xl*area(k)*rho/2.0

do 100 n=1,nj

nn=3*n

b(nn)=w(n)

b(nn-1)=v(n)

100    b(nn-2)=u(n)

do 500 n=1,nj

nn=3*n

k=kode(n)

if(k.eq.0) goto 500

if(k-100.lt.0) goto 300

a(nn-2,1)=a(nn-2,1)+1.0e11

b(nn-2)=b(nn-2)+1.0e11*u(n)

k=k-100

300    if(k-10.lt.0) goto 400

a(nn-1,1)=a(nn-1,1)+1.0e11

b(nn-1)=b(nn-1)+1.0e11*v(n)

k=k-10

400    if(k.eq.0) goto 500

a(nn,1)=a(nn,1)+1.0e11

b(nn)=b(nn)+1.0e11*w(n)

500    continue

call band1

do 600 n=1,nj

nn=3*n

u(n)=b(nn-2)

v(n)=b(nn-1)

600    w(n)=b(nn)

write(iflag,2000)

write(iflag,2008) (n,u(n),v(n),w(n),n=1,nj)

write(4,'(i5,3e15.6)') (n,u(n),v(n),w(n),n=1,nj)

2000   format(////' Nodal displacements')

2008 format(///4x,' Joint',14x,'U',14x,'V',14x,'W'/(i10,3e15.6))

return

end

subroutine stress

common a(200,25),s(6,6),sgw(3,3),area(240),ymod(240),b(240),

*u(63),v(63),w(63),x(63),y(63),z(63),pgw(30),xgw(30),ygw(30),

*  zgw(30),hed(20),t(6),cosa(3),

* kode(63),nodi(240),nodj(240),nodgw(30),lm(6),

* agw,egw,rho,iflag,maxb,maxe,mb,neq,ne,ngw,nj

write(iflag,2010)

do 500 n=1,ne

i=nodi(n)

j=nodj(n)

dx=x(j)-x(i)

dy=y(j)-y(i)

dz=z(j)-z(i)

xl=sqrt(dx**2+dy**2+dz**2)

du=u(j)-u(i)

dv=v(j)-v(i)

dw=w(j)-w(i)

dl=dv*dy/xl+du*dx/xl+dw*dz/xl

sig=dl*ymod(n)/xl

p=area(n)*sig

500    write(iflag,2011)n,i,j,p,sig

do 600 i=1,ngw

nn=nodgw(i)

dx=xgw(i)-x(nn)

dy=ygw(i)-y(nn)

dz=zgw(i)-z(nn)

xl=sqrt(dx**2+dy**2+dz**2)

600    pgw(i)=-egw*agw/(xl**2)*(u(nn)*dx+v(nn)*dy+w(nn)*dz)

return

2010   format(////'El.No.',5x,'I',5x,'J',14x,'P',9x,'Stress')

2011   format(/3i6,2e15.6)

end

subroutine band1

common a(200,25),s(6,6),sgw(3,3),area(240),ymod(240),b(240),

*u(63),v(63),w(63),x(63),y(63),z(63),pgw(30),xgw(30),ygw(30),

*  zgw(30),hed(20),t(6),cosa(3),

* kode(63),nodi(240),nodj(240),nodgw(30),lm(6),

* agw,egw,rho,iflag,maxb,maxe,mm,nn,ne,ngw,nj

nl=nn-mm+1

nm=nn-1

mr=mm

do 100 n=1,nm

if(a(n,1).le.0.) goto 700

bn=b(n)

b(n)=bn/a(n,1)

if(n.gt.nl) mr=nn-n+1

do 100 l=2,mr

if(a(n,l).eq.0) goto 100

c=a(n,l)/a(n,1)

i=n+l-1

j=0

do 50 k=l,mr

j=j+1

50     a(i,j)=a(i,j)-c*a(n,k)

b(i)=b(i)-c*bn

a(n,l)=c

100    continue

i=nn

b(nn)=b(nn)/a(nn,1)

do 600 n=1,nm

i=i-1

if(n.lt.mm)mr=n+1

do 600 j=2,mr

k=i+j-1

600    b(i)=b(i)-a(i,j)*b(k)

return

700    write(iflag,2000)n

2000  format(' Zero or negative element on main diagonal of '

1 ' triangularized matrix for equation ',i5)

end

X

30

4 (0,0,96)

3 (0,144,192)

1 (144,96,96)

2 (0,144,0)

Y

Z

Fig. I.1

E   = 30,000

A1 = 8

A2 = 8

A3 = 12

Input File 3dtin.dat (see Fig. I.1)

Three dimensional truss

1

4,3

1,0,144,96,96,0,-30.0,0

2,111,0,144,0,0,0,0

2

3,111,0,144,192,0,0,0

4,111,0,0,96,0,0,0

1,1,2,8.0,30000.0

2,1,3,8.0,30000.0

3,1,4,12.0,30000.0

0,0,0

3

Output File t3dout1.dat

Three dimensional truss

Number of joints        =    4

Number of elements      =    3

Unit weight of material =     0.

Joint no.      Code            X                         Y                       Z                     RX                    RY                    RZ

1                0       .144000E+03    .960000E+02    .960000E+02    .000000E+00   -.300000E+02    .000000E+00

2              111     .000000E+00    .144000E+03    .000000E+00    .000000E+00    .000000E+00    .000000E+00

3              111     .000000E+00    .144000E+03    .192000E+03    .000000E+00    .000000E+00    .000000E+00

4              111     .000000E+00    .000000E+00    .960000E+02    .000000E+00    .000000E+00    .000000E+00

Element         I         J              A                       E

1              1         2    .800000E+01    .300000E+05

2              1         3    .800000E+01    .300000E+05

3              1         4    .120000E+02    .300000E+05

Nodal displacements

Joint              U                    V                     W

1    .469669E-02   -.382931E-01    .346945E-17

2    .150000E-09   -.500000E-10    .100000E-09

3    .150000E-09   -.500000E-10   -.100000E-09

4   -.300000E-09   -.200000E-09    .000000E+00

El.No.     I     J              P                     Stress

1        1     2    .187083E+02    .233854E+01

2        1     3    .187083E+02    .233854E+01

3        1     4   -.360555E+02   -.300463E+01