APPENDIX M

Program FRAMDYN for the Dynamic Analysis of Plane Rigid Frames

Introduction

The algorithm of this program is similar to that of the linear buckling analysis of trusses given in Appendix L. The program can calculate either lumped mass or consistent mass matrix depending on the requirement and the matrices are written in band form. For the stiffness and mass matrices, the eigenvalues (natural frequencies) are calculated using the Jacobi method of subspace iteration depending upon the degree of freedom. Eigenvalues calculated are squares of the natural frequencies and the eigenvector gives the mode shape. The matrices may be kept either in the condensed form or in the full form.

Input: The geometry and the topology are input to the program as given below.

Create input file as input.dat and give data as shown below.

(a)  title

(b) nj,ne,iflag,imass

nj- number of joints

ne- number of elements

iflag- 0 matrices are compressed

1 uncompressed form

imass- 0 for lumped mass

1 for consistent mass

(c) nodal data

n,kode,x,y

n- node number

k- kode depending on boundary conditions as explained in Appendix G

x,y- X and Y coordinates of the node n

(d) element data

m,nodi,nodj,area,ri,ymod,dens

m- element number

nodi- ith node of the element m

nodj- jth node of the element m

area- area of the member m

ri- moment of inertia of the member m

ymod- youngs modulus of the member m

dens- density of the material

The data must be terminated with 0

The program outputs in1.dat which is the input file for JACOBI to calculate

the eigenvalues and eigenvectors as done in Appendix L.

Input.dat (see Fig. M.1)

Dynamics of plane frame

1,111,0,0

2,0,0,4000.0

3,0,6000,4000

4,111,6000,0

1,1,2,11074,1.521 2e09,210000,7.8e-06

2,2,3,11074,1.5212e09,210000,7.8e-06

3,3,4,11074,1.5212e09,210000,7.8e-06

0

PROGRAM FRAMEDY.FOR

$debug

c	PROGRAM PLFRAM

c	program for the dynamic analysis of simple framed structures

common x(100),y(100),u(100),v(100),r(100),area(200),ri(200),

*ymod(200), b(300),a(300,30),hed(20),

* s(6,6),fef(6,200),fq(6),am(300,30),dens(200)

common ne,nj,neq,mb,kode(100),nodi(200),nodj(200),

* lm(6),in,io,maxe,iflag,kodec(100),id(100),imass

maxe=100

open( 1,file= 'input.dat',status='old')

open( 2,file= 'in1.dat',status='new')

c

c

call input

c

c

call stiff

c

if(imass.eq.0) call lumass

if(imass.eq.1) call comass

c

c

if(iflag.eq.1) goto 25

if(iflag.eq.0) goto 30

25 do 1000 i=1,neq

do 1000 j=1,mb

write(io,'(e15.7)') a(i,j)

1000 continue

do 1010 i=1,neq

do 1010 j=1,mb

1010 write(io,'(e15.7)') am(i,j)

write(*,*) 'no. of equations(nn)=',neq

write(*,*)'band width(mb)=',mb

nolem=neq*mb

write(*,*)'no. of elements in stiffness and mass matrix(nwi&mwj

*)=',nolem

goto 1030

30 kk=0

do 107 i=1,neq

if(kodec(i).eq.1) id(i)=0

if(kodec(i).eq.1) goto 108

kk=kk+1

id(i)=kk

108	 write(*,*) 'id(',i,')=',id(i)

107 continue

index=kk

write(*,1879) index

write(*,*)'band width(mb)=',mb

nolem=index*mb

write(*,*)'no. of elements in stiffness and mass matrix(nwi&mwj

*)after compressing=',nolem

1879 format('  no. of equations=',i5)

do 110 i=1,neq

ii=id(i)

ij=0

if(ii.eq.o) goto 110

do 157 j=1,mb

k=j+i-1

if(k.gt.neq) goto 158

jj=id(k)

if(jj.eq.0) goto 152

ij=ij+1

a(ii,ij)=a(i,j)

am(ii,ij)=am(i,j)

goto 157

158 ij1=ij+1

do 159 it=ij1,mb

a(ii,it)=0.0

am(ii,it)=0.0

159 continue

goto 110

152	if(index.le.mb) ij=ij+1

if(index.le.mb) a(ii,ij)=0.0

if(index.le.mb) am(ii,ij)=0.0

157 continue

110 continue

do 1701 i=1,index

do 1701 j=1,mb

1701 write(2,'(e15.7)') a(i,j)

do 1702 i=1,index

do 1702 j=1,mb

1702 write(2,'(e15.7)') am(i,j)

c

c

1030 continue

end

c

c

subroutine input

c

common x(100),y(100),u(100),v(100),r(100),area(200),ri(200),

*ymod(200), b(300),a(300,30),hed(20),

* s(6,6),fef(6,200),fq(6),am(300,30),dens(200)

common ne,nj,neq,mb,kode(100),nodi(200),nodj(200),

* lm(6),in,io,maxe,iflag,kodec(100),id(100),imass

in=1

io=2

njmax=60

nemax=80

mbmax=30

c

read(in,1000) hed,nj,ne ,iflag,imass

if(nj.gt.njmax.or.ne.gt.nemax) go to 999

write( io,2000) hed,nj,ne,iflag,imass

c

write( io,2001)

do 100 m=1,nj

read(in,1001)n,kode(n),x(n),y(n)

backspace 1

read(1,'(i5,2x,3i1)') n,(kodec(i),i=3*n-2,3*n)

write(*,'(i5,2x,3i1)') n,(kodec(i),i=3*n-2,3*n)

100  write( io,2002) n,kode(n),x(n),y(n)

c

write( io,2003)

ndif=0

do 200 n=1,ne

read(in,1002) m,nodi(m),nodj(m),area(m),ri(m),ymod(m),dens(m)

k=iabs(nodi(m)-nodj(m))

if(k.gt.ndif)ndif=k

200	 write( io,2004) m,nodi(m),nodj(m),area(m),ri(m),ymod(m),dens(m)

mb=3*(ndif+1)

neq=3*nj

write(*,*) 'no. of equations(nn)=',neq

write(*,*)'band width(mb)=',mb

nolem=neq*mb

write(*,*)'no. of elements in stiffness and mass matrix(nwi&mwj

*)=',nolem

c

520  return

c

999  write( io,1999)

stop

c

c	format statements

1000  format(20a4/4i5)

1001  format(2i5,2f12.0)

1002  format(3i5,4f12.0)

1100  format(2i5,6f10.0)

2000  format('1',20a4/'  **number of joints     =',i4/

Š     *                '  **number of elements   =',i4/

*                ' ******iflag*********    =',i4/

*                '********imass********    =',i4/)

2001 format(' echo check of joint data'/' joint kode',6x,'x',llx

*,'y')

2002  format(2i5,2e12.4)

2003  format('0',' echo check of element data'/' memb nodi nodj ',7x

*'area',7x,'inertia',6x,'modulus')

2004  format(3i5,4e15.5,i5)

2200  format(3i5,6f11.4)

1999  format('  *** parameters exceed those permitted by program **')

end

subroutine stiff

c

c

common x(100),y(100),u(100),v(100),r(100),area(200),ri(200),

*ymod(200), b(300),a(300,30),hed(20),

* s(6,6),fef(6,200),fq(6),am(300,30),dens(200)

common ne,nj,neq,mb,kode(100),nodi(200),nodj(200),

* lm(6),in,io,maxe,iflag,kodec(100),id(100),imass

dimension t(3,6),cm(3,3),temp(3,6)

data t(1,6),t(2,3),t(3,3),t(3,6),cm(1,3),cm(2,3),cm(3,1),

*cm(3,2)/8*0.0/,t(1,3),t(2,6)/2*1.0/

c

c	initialize stiffness matrix

do 50 j=1,mb

do 50 i=1,neq

50 a(i,j)=0.0

do 500 m=1,ne

c

c	form member stiffness matrix

i=nodi(m)

j=nodj(m)

dx=x(j)-x(i)

dy=y(j)-y(i)

xl=sqrt(dx**2+dy**2)

xli=1./xl

alp=ymod(m)*ri(m)*xli

beta=ymod(m)*area(m)*xli

co=dx*xli

si=dy*xli

cl=co*xli

sl=si*xli

t(1,1)=-sl

t(1,2)=cl

t(1,4)=sl

t(1,5)=-cl

t(2,1)=-sl

t(2,2)=cl

t(2,4)=sl

t(2,5)=-cl

t(3,1)=-co

t(3,2)=-si

t(3,4)=co

t(3,5)=si

Š	 cm(3,3)=beta

110  cm(1,1)=4.*alp

cm(1,2)=2.*alp

cm(2,1)=cm(1,2)

cm(2,2)=cm(1,1)

c

200 do 300 l=1,6

do 300 k=1,3

d=0.0

do 250 n=1,3

250   d=d+cm(k,n)*t(n,l)

300  temp(k,l)=d

c

do 400 l=1,6

do 400 k=l,6

d=0.0

do 350 n=1,3

350 d=d+t(n,l)*temp(n,k)

s(l,k)=d

400 s(k,l)=d

do 1 i=1,6

write(*,*)(s(i,j),j=1,6)

1   continue

c	assemble stiffness matrix

lm(3)=3*nodi(m)

lm(2)=lm(3)-1

lm(1)=lm(3)-2

lm(6)=3*nodj(m)

lm(5)=lm(6)-1

lm(4)=lm(6)-2

do 450 i=1,6

ii=lm(i)

do 450 j=1,6

jj=lm(j)-ii+1

if(jj.le.0) goto 450

a(ii,jj)=a(ii,jj)+s(i,j)

450 continue

500 continue

c

c

do 600 n=1,nj

if(kode(n).eq.0) goto 600

k=kode(n)

ex=0.0

ey=0.0

er=0.0

if((k-100).lt.0) goto 520

ex=1.0e20

k=k-100

520 if (k-10.lt.0) goto 540

ey=1.0e20

k=k-10

540 if (k.eq.0) goto 550

er=1.0e20

550 nn=3*n

Š	 a(nn,1)=a(nn,1)+er

a(nn-1,1)=a(nn-1,1)+ey

a(nn-2,1)=a(nn-2,1)+ex

600 continue

c

c

c

c

c

return

end

subroutine lumass

c

c

common x(100),y(100),u(100),v(100),r(100),area(200),ri(200),

*ymod(200), b(300),a(300,30),hed(20),

* s(6,6),fef(6,200),fq(6),am(300,30),dens(200)

common ne,nj,neq,mb,kode(100),nodi(200),nodj(200),

* lm(6),in,io,maxe,iflag,kodec(100),id(100),imass

dimension t(6,6),cm(6,6),temp(6,6)

data t(1,3),t(1,4),t(1,5),t(1,6),t(2,3),t(2,4),t(2,5),t(2,6),

1     t(3,1),t(3,2),t(3,4),t(3,5),t(3,6),t(4,1),t(4,2),t(4,3),

2     t(4,6),t(5,1),t(5,2),t(5,3),t(5,6),t(6,1),t(6,2),t(6,3),

3     t(6,4),t(6,5)/26*0.0/,t(3,3),t(6,6)/2*1.0/

c

c	initialize stiffness matrix

do 50 j=1,mb

do 50 i=1,neq

50 am(i,j)=0.0

do 500 m=1,ne

c

c	form member stiffness matrix

i=nodi(m)

j=nodj(m)

dx=x(j)-x(i)

dy=y(j)-y(i)

xl=sqrt(dx**2+dy**2)

xli=1./xl

beta=dens(m)*area(m)*xl*0.5

co=dx*xli

si=dy*xli

cl=co*xli

sl=si*xli

t(1,1)=co

t(1,2)=si

t(2,1)=-si

t(2,2)=co

t(4,4)=co

t(4,5)=si

t(5,4)=-si

t(5,5)=co

do 60 i=1,6

do 60 j=1,6

60 cm(i,j)=0.0

cm(1,1)=beta

cm(2,2)=beta

cm(4,4)=beta

cm(5,5)=beta

c

200 do 300 l=1,6

do 300 k=1,6

d=0.0

do 250 n=1,6

250   d=d+cm(k,n)*t(n,l)

300  temp(k,l)=d

c

do 400 l=1,6

do 400 k=l,6

d=0.0

do 350 n=1,6

350 d=d+t(n,l)*temp(n,k)

s(l,k)=d

400 s(k,l)=d

c	assemble stiffness matrix

lm(3)=3*nodi(m)

lm(2)=lm(3)-1

lm(1)=lm(3)-2

lm(6)=3*nodj(m)

lm(5)=lm(6)-1

lm(4)=lm(6)-2

do 450 i=1,6

ii=lm(i)

do 450 j=1,6

jj=lm(j)-ii+1

if(jj.le.0) goto 450

am(ii,jj)=am(ii,jj)+s(i,j)

450 continue

500 continue

c

c

do 600 n=1,nj

if(kode(n).eq.0) goto 600

k=kode(n)

ex=0.0

ey=0.0

er=0.0

if((k-100).lt.0) goto 520

ex=1.0e20

k=k-100

520 if (k-10.lt.0) goto 540

ey=1.0e20

k=k-10

540 if (k.eq.0) go to 550

er=1.0e20

550 nn=3*n

am(nn,1)=am(nn,1)+er

am(nn-1,1)=am(nn-1,1)+ey

am(nn-2,1)=am(nn-2,1)+ex

600 continue

c

c

return

end

subroutine comass

c

c

common x(100),y(100),u(100),v(100),r(100),area(200),ri(200),

*ymod(200), b(300),a(300,30),hed(20),

* s(6,6),fef(6,200),fq(6),am(300,30),dens(200)

common ne,nj,neq,mb,kode(100),nodi(200),nodj(200),

* lm(6),in,io,maxe,iflag,kodec(100),id(100),imass

dimension t(6,6),cm(6,6),temp(6,6)

c

c	initialize stiffness matrix

do 50 j=1,mb

do 50 i=1,neq

50 am(i,j)=0.0

do 500 m=1,ne

c

do 950 ik=1,6

do 950 jk=1,6

950  t(ik,jk)=0.0

c	form member stiffness matrix

i=nodi(m)

j=nodj(m)

dx=x(j)-x(i)

dy=y(j)-y(i)

xl=sqrt(dx**2+dy**2)

xli=1./xl

beta=dens(m)*area(m)*xl/(420.0)

co=dx*xli

si=dy*xli

cl=co*xli

sl=si*xli

t(1,1)=co

t(1,2)=si

t(2,1)=-si

t(2,2)=co

t(4,4)=co

t(4,5)=si

t(5,4)=-si

t(5,5)=co

t(3,3)=1.0

t(6,6)=1.0

do 60 iii=1,6

do 60 jjj=1,6

60 cm(iii,jjj)=0.0

cm(1,1)=140.0*beta

cm(1,4)=70.0*beta

cm(2,2)=156.0*beta

cm(2,3)=22.0*xl*beta

cm(2,5)=54.0*beta

cm(2,6)=-13.0*xl*beta

cm(3,3)=4.0*xl*xl*beta

cm(3,5)=13.0*xl*beta

cm(3,6)=-3.0*xl*xl*beta

cm(4,4)=140.0*beta

cm(5,5)=156.0*beta

cm(5,6)=-22.0*xl*beta

cm(6,6)=4.0*xl*xl*beta

do 70 il=1,6

do 70 jl=il,6

70  cm(jl,il)=cm(il,jl)

c

write(*,*)' consistent mass matrix'

do 1 i=1,6

write(*,*)(cm(i,j),j=1,6)

1   continue

200 do 300 l=1,6

do 300 k=1,6

d=0.0

do 250 n=1,6

250   d=d+cm(k,n)*t(n,l)

300  temp(k,l)=d

c

do 400 l=1,6

do 400 k=l,6

d=0.0

do 350 n=1,6

350 d=d+t(n,l)*temp(n,k)

s(l,k)=d

400 s(k,l)=d

c	assemble stiffness matrix

lm(3)=3*nodi(m)

lm(2)=lm(3)-1

lm(1)=lm(3)-2

lm(6)=3*nodj(m)

lm(5)=lm(6)-1

lm(4)=lm(6)-2

do 450 i=1,6

ii=lm(i)

do 450 j=1,6

jj=lm(j)-ii+1

if(jj.le.0) goto 450

am(ii,jj)=am(ii,jj)+s(i,j)

450 continue

500 continue

c

c

c

c

do 600 n=1,nj

if(kode(n).eq.0) goto 600

k=kode(n)

ex=0.0

ey=0.0

er=0.0

if((k-100).lt.0) goto 520

ex=1.0e20

k=k-100

520 if (k-10.lt.0) goto 540

ey=1.0e20

k=k-10

540 if (k.eq.0) goto 550

er=1.0e20

550 nn=3*n

am(nn,1)=am(nn,1)+er

am(nn-1,1)=am(nn-1,1)+ey

am(nn-2,1)=am(nn-2,1)+ex

600 continue

c

c

c

c

c

return

end

Input File for Jacobi  in1.dat

The output file in1.dat of framedy.for will be the input file for JACOBI.

Delete all the data except the matrix elements.  In the first line give the data for

nn,ifss,ifpr,iout,mb,msmax as explained in Appendix L.

Output out1.dat

Sweep number in Jacobi =  10

Current eigenvalues in Jacobi are

.1000000E+01   .1000000E+01   .1000000E+01   .1012750E+03

.7745731E+03   .9067657E+04

.3556724E+04   .2520023E+04   .5609040E+04   .1000000E+01

.1000000E+01   .1000000E+01

Least eigenvalue=101.27

Square of the natural frequency =101.27

Natural circular frequency=10.06 rad/s