# Inspect and Replace Camshaft Drives

Name 							Station				Date

NATEF Correlation

This LAB SHEET addresses the following NATEF task:

B. 12.	Inspect camshaft drives (including gear wear and backlash, sprocket and chain wear); replace as necessary.

Describe the engine being worked on:

Year					Make					Model

VIN					Engine type and size

1. Disconnect the negative battery cable.

2. Remove the covers protecting the camshaft drive gears.

3. Check for proper alignment of the timing marks on the timing gears.

4. Remove the timing gears according to the service manual.

5. Clean the timing gears.

6. Inspect the gears for wear or cracks, and replace necessary parts.

7. After replacing any worn or damaged gears and drives, set the timing marks according to the service manual.

8. Rotate the engine through two complete rotations of the crankshaft. Check to see if the timing marks on the gears are still in proper alignment.

9. Reinstall all covers removed for access.

10. Reconnect the negative battery cable and start the engine.

11. Set the ignition timing to the manufacturer's specifications.

Problems Encountered

Instructor's Comments