Installing an Engine In a RWD Vehicle

Name							Station				Date

NATEF Correlation

This LAB SHEET addresses the following NATEF task:

A.12. Reinstall engine (rear-wheel drive).

Describe the vehicle being worked on:

Year				Make						Model

VIN 				Engine type and size

Procedure:

Connect the engine to a sling and then connect the sling to the crane. What did you connect the sling to?

Place a transmission jack under the transmission to hold it in position.

Slowly lower the engine into the engine compartment. Guide the engine around all wires and hoses to make sure nothing gets damaged. Did you have any problems doing this?

As the engine approaches its position in the engine compartment, align the engine to the input shaft of the transmission or the torque converter hub into the front pump. Did you have any problems doing this?

Carefully wiggle the engine until the input shaft slides through clutch disc splines or the torque converter seats fully into the transmission.

Install and tighten the transmission to engine bolts.

Start the engine mount bolts into their bores; you may need to wiggle the engine some to do this.

Once the mount bolts are in place, tighten them and remove the transmission jack and engine sling.

Raise the vehicle to a good working height.

While working under the vehicle, install all remaining engine and transmission mounts.

Connect the exhaust manifold to the exhaust system. Was a new gasket required? Did you have any problems doing this?

Install any heat shields that were removed when the engine was removed.

Reconnect the fuel line from the fuel tank to the engine. Where is this connection made?

Lower the vehicle so you can work under the hood.

Connect any remaining disconnected fuel lines.

Connect heating system hoses. Did you need to replace any? Why?

Connect the engine ground strap.

Connect all electrical connectors and wires.

Connect all vacuum and other hoses.

Connect the throttle linkage and adjust it according to the manufacturer's procedures. Did you need to adjust the linkage? If so, how did you do it?

Install the radiator and the cooling fan(s).

Connect the rest of the hoses for the cooling system. Did you need to replace any of these? Why?

Install the air induction system.

Connect any remaining items.

Install the battery and connect the battery cables. Which cable did you connect first? Why?

Fill the radiator with coolant and visually check for leaks. Did you find any?

27.If the engine doesn't have oil in it already, add the specified amount of the proper type of oil.

Prime the oil pump of the engine. How did you do that?

Prepare the engine for startup. What did you need to do?

Install and align the hood.

Instructor's Comments