Installing an Engine Into a FWD Vehicle

Name 						Station					Date

NATEF Correlation This LAB SHEET addresses the following NATEF task:

A.11. Reinstall engine (front-wheel drive).

Describe the vehicle being worked on:

Year						Make					Model

VIN 						Engine type and size

Procedure:

Was the engine removed from the top or underneath?

If the engine will be installed through the top:

Connect the engine to a sling and then connect the sling to the crane. What did you connect the sling to?

Slowly lower the engine into the engine compartment. Guide the engine around all wires and hoses to make sure nothing gets damaged. Did you have any problems doing this?

As the engine approaches its position in the engine compartment, align the engine and transmission mounts. Did you have any problems doing this?

Once the mounts are aligned, lower the engine so you can install the bolts into the mounts.

Raise the vehicle to a good working height.

If the engine will be installed from under the car.

Install the engine onto the engine cradle and dolly. What did you use to secure the engine onto the dolly?

Lift the vehicle on a hoist or lift.

Position the engine under the vehicle.

Slowly lower the vehicle over the engine while guiding all wires and hoses out of the way. Did you have any problems doing this?

As the vehicle gets close to the engine, align the engine and transmission mounts. Did you have any problems doing this?

Once the mounts are aligned, lower the vehicle so you can install the bolts into the mounts.

Raise the vehicle to a good working height.

After the engine is resting on its mounts:

1. While working under the vehicle, install the axle shafts.

Install the remaining engine and transaxle mounts and braces. What do these connect to?

Connect the exhaust manifold to the exhaust system. Was a new gasket required? Did you have any problems doing this?

Install any heat shields that were removed when the engine was removed.

Connect all linkages to the transmission. What linkages were there?

Connect all lines, hoses, and electrical wiring to the transmission. What did you need to connect?

Reconnect all suspension and steering parts that were disconnected or removed. After you do this, will the vehicle need a front wheel alignment? Why?

Install the wheels and tires. What is the torque spec for the wheel lugs?

Tighten the axle hub nuts. What is the torque spec for these nuts?

Reconnect the fuel line from the fuel tank to the engine. Where is this connection made?

Lower the vehicle so you can work under the hood.

Connect any remaining disconnected fuel lines.

Connect heating system hoses. Did you need to replace any? Why?

Connect the engine ground strap.

Connect all electrical connectors and wires.

Connect all vacuum and other hoses.

Connect the throttle linkage and adjust it according to manufacturer's procedures. Did you need to adjust the linkage? If so, how did you do it?

Install the radiator and the cooling fan(s).

Connect the rest of the hoses for the cooling system. Did you need to replace any of these? Why?

Install the air induction system.

Connect any remaining items.

Install the battery and connect the battery cables. Which cable did you connect first? Why?

Fill the radiator with coolant and visually check for leaks. Did you find any?

If the engine doesn't have oil in it already, add the specified amount of the proper type of oil.

Prime the oil pump of the engine. How did you do that?

Prepare the engine for startup. What did you need to do?

Instructor's Comments