Measure Cylinder Bore

Name							Station				Date

NATEF Correlation

This LAB SHEET addresses the following NATEF task:

C.4. Inspect and measure cylinder walls for damage and wear; determine necessary action.

Describe the engine being worked on:

Year				Make					Model

VIN 						Engine type and size

1. Using the appropriate service manual, look up the specifications for standard bore size, tolerances for out-of-roundness, and taper. Record these specifications on the Report Sheet for Measuring Cylinder Bore.

2. Release the lock screw at the end of the handle of the telescoping gauge. Compress the plungers and, using the lock screw, secure them in the retracted position. Place the telescoping gauge into the bore, in an area below ring travel and 90 degrees to the crankshaft.

3. Release the plungers by loosening the lock screw. Allow the plungers to expand until they contact the bore walls. Rock the telescoping gauge back and forth and side to side to check for the correct resistance. Lock the plungers into position using the lock screw. Carefully retract the telescoping gauge from the bore.

4. Use an outside micrometer to measure the distance between the two plunger faces. Record the reading obtained for each cylinder on the Report Sheet for Measuring Cylinder Bore.

5. Measure the diameter of the cylinder at the highest point of piston ring travel. The difference between this measurement and the diameter below ring travel is the amount the cylinder is tapered. Record the amount of taper for each cylinder on the Report Sheet for Measuring Cylinder Bore.

6. Measure for cylinder out-of-roundness by first measuring the bore at a point parallel to the piston pin, then at right angles to the piston. The difference between these measurements is the amount of out-of-roundness. Measure for out-of-roundness at three locations in each bore (top, middle, and bottom). Record the worst condition for each cylinder on the Report Sheet for Measuring Cylinder Bore.

7. Perform a visual inspection of the block and record your findings on the Report Sheet for Measuring Cylinder Bore.

Problems Encountered:

Instructor's Comments

Name					Station					Date

| REPORT SHEET FOR MEASURING CYLINDER BORE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cylinder No. |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| Standard bore size |  |  |  |  |  |  |  |  |  |
| Actual bore size |  |  |  |  |  |  |  |  |  |
| Taper limits |  |  |  |  |  |  |  |  |  |
| Actual taper |  |  |  |  |  |  |  |  |  |
| Out-of-round limits |  |  |  |  |  |  |  |  |  |
| Actual out-of-roundness |  |  |  |  |  |  |  |  |  |
|  | Visual Inspection |  |  |  |  |  |  |  |  |
| Cracks and scores | OK |  |  |  | Not serviceable |  |  |  |  |
| Core-hole plugs | OK |  |  |  | Not serviceable |  |  |  |  |
| Lifter bores | OK |  |  |  | Not serviceable |  |  |  |  |
| Water jackets | OK |  |  |  | Not serviceable |  |  |  |  |
| Machined surfaces | OK |  |  |  | Not serviceable |  |  |  |  |
| Bolt holes and threads | OK |  |  |  | Not serviceable |  |  |  |  |
| Oil galleries | OK |  |  |  | Not serviceable |  |  |  |  |
| Oil passages | OK |  |  |  | Not serviceable |  |  |  |  |
| Conclusions and Recommendations |  |  |  |  |  |  |  |  |  |