Measure Engine Oil Pressure

Name ___________________ Station ___________ Date _______

NATEF Correlation

This LAB SHEET addresses the following NATEF task:

D.I. Perform oil pressure tests; determine necessary action

Describe the vehicle being worked on:

Year _______ Make ________________ Model _____________

VIN ________________ Engine type and size

Describe general condition:

PROCEDURE

1. Look up the oil pressure specifications for this engine in the appropriate service manual:

Oil pressure

Engine speed

2. Locate the oil-pressure sending unit; usually it is on the lower side of the engine block. Disconnect the wire from the sending unit and use an openend wrench to remove the sender.

3. Tighten the oil-pressure test gauge into the hole in the block where the sender was removed. Use an adapter fitting, if necessary, to make the connection.

4. Check the engine's oil level and fill, if required.

WARNING: Be extremely careful when working near a running engine. Always wear safety goggles or glasses with side shields when working around moving machinery and be sure that your clothing is not loose.

5. Start the engine and observe the pressure reading on the gauge. Make sure the engine speed is set to specifications for testing pressure. If necessary, use a tachometer and adjust the engine idle speed. Record the measured oil pressure, then turn the engine off.

6. Is your measurement below specifications?

7. Consult the appropriate service manual and list your conclusions and recommendations.

8. Remove the test gauge and adapter fitting. Reinstall the oil pressure sender and connect the wire. Start the engine and check for leaks.