Remove Cylinder Ring Ridge

Name						Station					Date

NATEF Correlation

This LAB SHEET addresses the following NATEF task:

C.4. Remove cylinder wall ridges.

Describe the vehicle being worked on:

Year					Make					Model

VIN 					Engine type and size

1. Rotate each piston to bottom dead center and inspect the cylinder to see if completed there is a ring ridge.   Use your fingernail to check for a ridge. If it catches on the cylinder wall, the ridge must be removed.

2. Select the correct-size ridge reamer and position it in the cylinder with the piston at bottom dead center. Adjust the cutter against the cylinder walls according to the instructions furnished with the tool.

3. Use a wrench to turn the tool in a clockwise direction.

WARNING: Always wear safety goggles or glasses with side shields when completing this task.

Rotate the tool around the cylinder until the ridge is removed. Be careful not to remove more than the ridge.

4. Repeat this operation in each of the other cylinders.

5. Use an oily rag to remove the cuttings from each cylinder.

Problems Encountered