Repair and Replace Damaged Threads

Name						Station					Date

NATEF Correlation

This LAB SHEET addresses the following NATEF task:

C.3. Inspect internal and external threads; restore as needed (includes installing thread inserts).

1. Using a thread gauge, determine the proper thread pattern of the damaged threads.

2. Thoroughly clean the threads to be repaired.

3. Select the proper size tap to repair internal threads or die to repair external threads.

4. Align the repair tool with the damaged threads, and slowly rotate the tool in the direction of the threads. This tool will clean and straighten the damaged threads.

5. When you have reached the end of the damaged threads, remove the tool by reversing its rotation.

CAUTION: Use a light oil to enable the taps and dies to cut easily and cleanly.

When internal threads in an object are damaged and cannot be repaired, they are sometimes replaced with a heli-coil.

1. Select the proper size heli-coil kit and use the drill bit size recommended to drill out the hole.

2. Use the tap furnished to thread the oversized hole.

3. Use the provided tool to install the new thread kit.

Problems Encountered