Using a Vacuum Gauge

Name ___________________ Station ___________ Date ___________

NATEF Correlation

This LAB SHEET addresses the following NATEF task:

A.5. Perform engine vacuum tests; determine necessary action.

Describe the vehicle being worked on:

Year _______ Make ________________ Model ________________

VIN ________________ Engine type and size

Describe the general operating condition:

Give a definition of vacuum

PROCEDURE

1. Carefully look over the engine's intake manifold to identify vacuum hoses. Select one that is small and easily accessible. DO NOT disconnect it until you have the approval of your instructor. Also make sure the hose is below the throttle plate assembly.

2. With the engine off, disconnect the selected hose. Connect the Tee fitting to the end of the hose and connect another hose from the Tee fitting to the place where the vacuum hose was originally connected.

3. Connect the vacuum gauge to the remaining connection at the Tee fitting.

4. Start the engine and observe the vacuum gauge.

5. Describe the gauge reading and the action of the needle.

6. Quickly open the throttle and allow it to quickly close.

7. Describe the gauge reading and the action of the needle.

8. Turn off the engine and disconnect the vacuum gauge and hoses. Then reconnect the engine's hose to its appropriate connector.

Summarize what you observed.