ViewDatapath release 1.0 -- 7/6/94

Installation

============

1. Licensing.  ViewDatapath requires a 5.2 license group containing

product 518.

2. Installing menus.  If you already have a vdrawus.vs file in the

WDIR path, add the following line:

load ("vdpath.vs");

If you do not have a vdrawus.vs file, use a text editor to create

the file, enter the line shown above, and place it in one of your WDIR

directories.

3. Accessing the symbol library.  Add the following line to your viewdraw.ini

search order:

DIR [r] powerview_install_dir/lib/vdpath (vdpath)

Replace the text "powerview_install_dir" with the pathname of the

directory in which Powerview has been installed.  Note that the

library alias "vdpath" is required.

Known bugs

==========

1. If you type in a value to the slider widgets used to control the bit

width, and if the value is out of range, you may get unexpected results

without a warning message.  The text you type in is truncated to fit

the range; entering "100" into a slider with a maximum value of 20 will

produce the value "10."  Extra characters are ignored.  Scopus #5251.

Workaround: use the slider, or make sure the value you type in is in range.

2. It is not possible to automatically vary the delay values for different

symbols.  While this could be done with an attribute and a generic for

Viewsim style VHDL, neither ViewSim/VHDL style VHDL nor Synopsys style

VHDL permit this.

Workaround: edit the automatically created VHDL to insert the desired delay.

3. Cells which have a two dimensional input -- multiplexor and combinational

gates -- can not be simulated in Viewsim/VHDL.  Only subset VHDL may be

used.  This is due to a shortcoming in the LPM standard itself: the required

name of the input pins uses the form DATAi_j, which implies a bit vector

whose name is DATAi_.  Trailing underscores are illegal in VHDL identifiers.

Workaround: use subset VHDL for these modules.