ViewSim, ViewVHDL, and ViewSim/VHDL 5.3.1 Release Notes

Overview

ViewSim is a high-capacity, full-timing simulator and (optional) debugger that

meets the needs of design in the development of electronic systems and

integrated circuits. ViewSim provides a graphical user interface for mixed-mode

simulation of switch-level transistors, gates, and functional and behavioral blocks.

ViewSim/VHDL integrates the Vantage VHDL simulation kernel with the ViewSim

gate-level simulator and the VHDL source-level debuggers. With ViewSim/VHDL,

you can analyze and concurrently simulate designs containing IEEE-1076 compliant

ViewVHDL is a VHDL language simulator. It contains all the features of

ViewSim/VHDL except that it cannot simulate ViewSim gates.

These Release Notes outline the ViewSim, ViewVHDL, and ViewSim/VHDL 5.3.1

features and enhancements, as well as additional information regarding ViewSim,

ViewVHDL, and ViewSim/VHDL.

Platform Support

ViewSim, ViewVHDL, and ViewSim/VHDL 5.3.1 platform support includes the

following:

Sun 4/SPARCstation running X11, or OpenWindows, under SunOS V4.1.3 or

V5 (Solaris 2.3 or 2.4)

IBM RISC System/6000 running X11 under AIX V3.2.5

Hewlett-Packard 9000 Series 700 running X11, or HP VUE, under HP-UX V9.05

To simulate VHDL designs under HP-UX V9.05 you need the unbundled Hewlett-Packard

C compiler as well as patches for that C compiler and linker. You obtain the

compiler and patches from Hewlett-Packard.

Media Distribution & Installation

CD-ROM is the default distribution media for all platforms.  (Tape distribution

is available by request.) You install ViewSim, ViewVHDL, and ViewSim/VHDL from

the CD to local disk. On-line documentation (ViewDoc) is also the default publication

set for all platforms.  ViewDoc and the libraries are not directly accessible

from the CD.

Licensing Changes

ViewSim/VHDL and ViewVHDL licenses now include a license for the LMC Swift

Model interface.

You have the option of purchasing a license for:

The Vantage CodePerfect tool that generates reports on source code line executions

and signal or variable toggles

The Vantage STYX model interface that is a C library interface

ViewSim Product Enhancements

The ViewSim product enhancements include:

Saber/ViewSim integration

For information on Saber/ViewSim integration see the Saber/ViewSim STI User's Guide.

LMC Swift SmartModels interface support

For information on Swift SmartModels interface support See Appendix D Using

SmartModels in the ViewSim Reference Manual.

Note: If you currently use LAI SmartModels and have customized your symbol

library, both Viewlogic and LMC have tools to simplify the migration from the

old LAI interface to the new SWIFT interface. When you choose to migrate contact

your Viewlogic Application Engineer or LMC for more information.

The second phase of strength handling that includes:

Displaying the circuit driver value

See the display => info command in Chapter 6 ViewSim Command Reference in the

ViewSim Reference Manual.

The reportx option to the defaults command warning you when a signal is unstable

or in an unknown state

You can use this command and option to flag bus contention.

See the Defaults Reportx command in  Chapter 6 ViewSim Command Reference in

the ViewSim Reference Manual.

The if command now works dynamically during simulation

See the If command in  Chapter 6 ViewSim Command Reference in the ViewSim Reference

Manual.

Reduced the time required to initialize data files associated with large memories

that use the ViewSim Builtin memory models.

Reduced the time required to read the .vsm file.

ViewSim/VHDL and ViewVHDL Product Enhancements

The ViewSim/VHDL and ViewVHDL product enhancements include:

The latest Vantage 5.2 analyzer and simulation kernel that gives you substantial

VHDL simulation performance improvements

VHDL to VHDL direct connections for VHDL nets, called Vantage_only nets, in

schematic based designs

Vantage_only nets support all VHDL data types and simulate much faster than

other ViewDraw nets.

See Using Vantage_only Nets in Chapter 3 ViewSim/VHDL Analysis and Simulation

in the ViewSim/VHDL User's Guide.

Support for the Vantage STYX model interface that is a C interface library

See Appendix E Using The Vantage Styx C Model Interface Library  in the

ViewSim/VHDL User's Guide.

Support for accessing elements of arrays and composites in the user interface

and debugging commands

See Accessing The Elements of Composite Types in Chapter 3 ViewSim/VHDL Analysis

and Simulation in the ViewSim/VHDL User's Guide.

Different colors for keywords and comments in the source window

For information on the colors in the source window see The Source Window in

Chapter 5 The ViewSim Debugger in the ViewSim Reference Manual.

Access to the Vantage CodePerfect tool to report on VHDL source code line executions

and VHDL signal or variable toggles

See Appendix G Using The Vantage CodePerfect Tool in the ViewSim/VHDL User's

Guide.

When you enter an interactive state using a break command, then all simulators

are synchronized at that time and ViewSim/VHDL executes all commands waiting

or applied through a command file are executed at that time. (Previously the

simulation would continue without looking at the commands just read from the

command file).

The VHDL_CONFIGURATION attribute is not required any more. However if you attach

a VHDL_CONFIGURATION attribute, ViewSim/VHDL uses it and that VHDL_CONFIGURATION

attribute will have higher precedence over the VHDL attribute. If you don't

attach VHDL_CONFIGURATION  attribute, but have attached only the VHDL attribute,

ViewSim/VHDL first looks for a configuration with that attribute value. If

it finds one, it uses that attribute to get the entity or architecture. If

the configuration is not found, ViewSim/VHDL assumes that the given name is

an entity. Thus you'll have one less attribute to worry about.

NOTE: If you have pre 5.3.1 ViewSim/VHDL you need VHDL_CONFIGURATION and if

both VHDL_CONFIGURATION and VHDL are present

VHDL takes precedence over VHDL_CONFIGURATION.

NOTE: the ViewDraw level =>push command could not be modified in time for

5.3.1 release. Hence in order for this to work, you need the VHDL_CONFIGURATION

attribute. Also if both  VHDL_CONFIGURATION and VHDL are present VHDL takes

precedence over VHDL_CONFIGURATION.

NOTE: If you don't want to get into this confusion, we recommend not attaching

both VHDL_CONFIGURATION and VHDL to the same instance or symbol.

Compatibility Of Designs Analyzed In Previous Releases

Viewsim/VHDL 5.3.1 comes with four versions of Vantage libraries. Some of these

versions are not compatible with previous library versions so you must re-analyze

your designs if you install one of these versions. Some of these versions are

optimized to increase simulation performance. The contents, optimization, and

re-analysis requirements of these versions are in the following table.

Library Version    Library Contents and Optimization      Re-analysis Requirements

libs.04            Highly optimized Vantage and IEEE      Must re-analyze

libraries

libs.dft           Regular libraries                      Re-analysis not required

libs_syn.04        Highly optimized Vantage, IEEE,        Must re-analyze

and Synopsys libraries

libs_syn.dft       Regular Vantage, IEEE,                 Must re-analyze

and Synopsys libraries

Install libs.04 or libs_syn.04 to increase simulation performance.

Install libs.dft to avoid re-analyzing your designs.

Disabling Vantage_only Nets

Nets or busses that connect only to Vantage VHDL models, are called Vantage_only

nets. Vantage_only nets support all VHDL data types and simulate much faster

than other ViewDraw nets. However the vector command does not work with these

nets.

If Vantage_only nets present problems during simulation, for example if your

simulation command files include vector commands for nets that now meet the

rules for Vantage_only nets, you can disable the Vantage_only net feature for

all nets that meet these rules or disable the feature for individual nets.

To disable the Vantage_only net feature for all nets you can:

Enter the vannetsoff command in your viewsim.ini file

Set the vlDisableVhdlNetSupport 1 UNIX environment variable before you invoke

ViewSim/VHDL

To use the environment variable enter:

setenv vlDisableVhdlNetSupport 1

To enable Vantage_only nets after you have disabled them with this environment

variable, enter the following before you invoke ViewSim/VHDL:

unsetenv vlDisableVhdlNetSupport

To disable the Vantage_only net feature for individual nets  assign the

VANTAGE_ONLY=OFF attribute to any of the ViewDraw components that is connected by

that net.