ViewFloorplanner Release 1.01 Installation Procedure

Loading the Software

The ViewFloorplanner software should be installed as a part of the powerview

installation process from a tape or a CD. It takes approximately 65Meg of disk

space for installing ViewFloorplanner software and tutorials.

Powerview 5.3.2 must be installed in order to run ViewFloorplanner.

User Setup

Make the following changes to your .cshrc file before running

ViewFloorplanner:

1.	Define the HEDA_HOME environment variable to where you installed

ViewFloorplanner.

setenv HEDA_HOME powerview_install_dir/viewfp

2.	Modify your LD_LIBRARY_PATH

setenv LD_LIBRARY_PATH

"$LD_LIBRARY_PATH":$HEDA_HOME/apps/itc/lib

3.	Modify your path to access ViewFloorplanner binaries

set path = ($path $HEDA_HOME/apps/nav/bin

$HEDA_HOME/apps/itc/bin)

4.	Source your .cshrc file for the changes in the first 3 steps to take affect.

source .cshrc

5.	Copy the place.config file to your user directory.

cp $HEDA_HOME/apps/nav/config/place.config

edanav.config

6.	To have a keyboard behavior of ViewFloorplanner similar to Viewdraw,

copy the hotkeys file to the project directory.

cp $HEDA_HOME/apps/nav/config/vendors/vlogic/hotkeys

project_dir/hotkeys

7.	In order to use ITC for cross-probing between Viewdraw and

ViewFloorplanner, add the following to your services file (/etc/services).

harris_itc 2710 /tcp

Note: This requires root privilege.

8.	Start the ITC daemon before running ViewFloorplanner.

$HEDA_HOME/apps/itc/bin/heda_start_itcd

9.	Add the following to your ~/.Xdefaults file:

place*foreground: 			black

place*background:			white

place*font:			Helvetica-bold

10.	Reset your X server resources

xrdb ~/.Xdefaults

11.	Add the following line to the vdrawus.vs file in the

powerview_installdir/standard area

load ("edanav.vs");

Licenses Required for running ViewFloorplanner

546 PCB Toolkit Application Program

631 ViewFloorplanner Design Explorer

632 ViewFloorplanner Partition compiler (optional)

633 ViewFloorplanner Parametric placement (optional)

634 ViewFloorplanner Timing Driven placement (optional)

635 ViewFloorplanner Congestion improver (optional)

636 ViewFloorplanner Advisor backplane (optional)

637 ViewFloorplanner Electrical Advisor I (optional)

638 ViewFloorplanner Thermal Advisor I (optional)

639 ViewFloorplanner Form Factor Advisor I (optional)

640 ViewFloorplanner Routability Advisor I (optional)

641 ViewFloorplanner Routability Advisor II (optional)

Invoking ViewFloorplanner

ViewFloorplanner will come up if you type the following:

viewfp -laf <motif | openlook>