ViewSyn 5.0.1 SPARC (5/12/95)

==============================

I. Bug fixes

-------------

20649     Problem with passing attributes in xblox

20486     Bad wir files created, execeeds 128 characters

19803     sis.exe and fpgasyn require dos extender in the path

19802     vsyn complains syntax error in synth/pla.out for designs wit

19754     Viewsyn crashes with problem with pla table.

19757     Error opening Pinorder file

19384     References to PRO Series in Workview PLUS version of ViewSyn

19262     vlpack: can't write block names containing [] characters

19141     Message "File creation error" displayed when vsyn calls subt

19137     vhdldes: "can't open . for write" when writing report file

19110     rebuilding SIS required change in interface with vsyn

19098     Windows: vsyn does not delete top.ref; All plats: vdes_cmd,

19097     After vhdm error, gnl not being cleaned up; next (clean) fil

19096     Clock enable counter failed with odd error message in vhdm

19095     iopad: addcum3 regression failed on pc, but not on workstati

19094     vlpack produced logic which could not be mapped into CLBs by

18966     Analysis of a different design causes ViewSyn to freeze

18965     Synthesis 2.2.3

18892     Errors from XBLOX appear after flattening one design

18862     Core dump when processing vhdl

18795     bad wir file generated

18739     Error opening Pinorder file

18444     vhdldes errors printed to stderr do not appear in output log

17896     The Viewsynthesis 2.4 installation disks are missing many sm

17568     Vsyn doesn't always perform iopad insertion when it should

17566     Scary error kicks you out of vhdldes when technology is chang

17048     xb_INC_DEC should be added to the list of XBLOX proceedures

ViewSyn 5.0 release notes (2/15/95)

I. New features

================

New Gui

-------

Powerview/UNIX GUI

~~~~~~~~~~~~~~~~~~~

o Hierarchy Browser

o Selection of Multiple vhdl files for compilation through Pick menu.

o Incremental work in progress state change information via icons

o Component Entity hierarchy dependency tree display

o Constraint setting via the GUI on selected blocks

o Links with viewlogic ITC

PC GUI

~~~~~~

o Access to Hierarchy Controller

Hierarchy Controller

--------------------

o Support for hierarchical designs

o Hierarchical manipulation dissolve, flatten, clone(uniquify)

o Hierarchical timing exported via SML library files per entity

o Accessible from command line as Vsyn command

o Selection of Multiple vhdl files for compilation.(Vsyn -vhdl *.vhd)

o Intelligent make means that one can incrementally compile

hierarchical designs.

o Support for partitioned and hierarchical ViewArchitect VHDL

VHDL improvements

------------------

o Support of Std_Logic built in

(first available in 2.4)

o Better performance on Ranges in large Case statements.

o Provision of a Std_logic VHDL simulation package for use with Vantage.

o Support for ViewSyn and ViewArchitect specific attributes

o Support for partitioned designs from ViewArchitect

o Increased Synopsys compatibility for sequential constructs

o Less restrictive syntax to allow more use of Clock enable inferencing

for xilinx

o Ignore Configuration and Generic statements

o Correct code now generated for certain "If-Else" statements

Improved Logic optimization

---------------------------

o New optimization engine gives improved results.

o Redundancy removal option to produce 100% testable logic.

Improved capacity

-----------------

o Hierarchy and improved logic optimization increase capacity from

5-10k to 100k +.

o single entity capacity (without hierarchy) improved.

Improved Mapping capability

---------------------------

o Fannout compensation based on re-powering and tree buffering.

o Hierarchical inter-block buffering added.

o Hierarchical timing and area information exported and reported.

XILINX Features

---------------

o Use of clock enable Flipflops to reduce CLBs used.

(First available 2.3.1, less restrictive syntax supported in this version)

o Mapping to iopads to use built in Flip-Flop

(First available 2.3.1)

o XILINX specific 1st generation specific CLB mapping to give accurate CLB

utilization plus reports on CLBs and logic levels

o Beta version of advanced CLB mapper II (no report available) available

from command line.

Links to layout

---------------

o Path Constraint information for Xilinx Place and Route passed to

XNF via the WIR file.

Improved support for busses

---------------------------

o Automatic Generation of Viewlogic symbols with bussed pins

o Generation of buses in WIR file for symbols with bussed pins.

o Viewgen support for bundled busses.

o Schematic displayed with busses.

Compatability Mode

------------------

o The old 2.x vhdldes command line is still supported.

(However hierarchy features,improved logic optimisation

and CLB mapper II cannot be accessed)

Improved constraint setting timing analysis

-------------------------------------------

o Timing analysis accounts for loads set on outputs.

Reporting

---------

o Improved timing report with critical path information

o Critical path Annotation on schematic via WIR file.

o Improved name traceability between register input and

output pins and original HDL signal names.

Documentation

----------------------

o The VHDL reference guide is updated and expanded into a style guide

o The ViewSyn user manual is updated for the new 5.0 GUI.

o An updated  Xilinx specific chapter is available.

Some Features available by using ViewArchitect ahead of ViewSyn

---------------------------------------------------------------

o Better coverage of Synopsys 3.x subset

o Operator inferencing

o Datapath component generation based on area /speed gives better

results and less cpu.

o Operator mapping to X-Blox components gives better

results for Xilinx designs.

o FSM recognition and optimization

o Resource sharing

(See ViewArchitect product documentation for full details)

Some Features available by using ViewFSM ahead of ViewSyn

---------------------------------------------------------

o State Diagram entry

o Redundant state and unreachable state removable

o Variety of State Encoding schemes

o Best in class FSM optimization.

(See ViewFSM product documentation for full details)

Libraries distributed with this release

---------------------------------------

Here's the list of synthesis libraries that are included in the distribution:

act1, act2, act3, altera, ami2un, attperf, cg21, cgaau, cmos5, cmos6,

dio, lsi100k, lsi10k, lca300k, scmos, vgt350, vgx700, vs700, vsc350, vsc370,

xc2000, xc3000, xc4000

For other libraries, please contact your local representative or vendor.

II.  Addendum

_______________________________________________________________________________

Bug ID:       17579               Reported:   Nov 09, 94

X-performance is not enabled in this release

Description:

------------

X-performance is not enabled in this release

Workaround:

-----------

None required

_______________________________________________________________________________

Bug ID:      17788                Reported:   Nov 23, 94

core dumps with NULL pin in SML file

Description:

------------

When there is an unconnected pin in a component such that the

sml for the component describes it as a NULL pin, a core dump

occurs while mapping the level containing an instance of

that component.

Workaround:

-----------

If a hierarchical component then flatten out the component

_______________________________________________________________________________

Bug ID:       18486               Reported:   Jan 11, 95

PC synthesis install path must be less than 10 characters

Description:

------------

If you install PC synthesis into a directory whose pathname is more than

ten characters, then XBLOX processing will fail with mysterious errors.

DOS has a limitation of 64 characters for the entire pathname for any

file.  Longer pathnames are simply truncated.  One of the pathnames

required for using the XBLOX package correctly is:

standard/vhdllibs/synth/xblox.pkg/vdes/work/xblox.pkg/tree.nam

which is 54 characters long.  If you install prosynthesis into a directory

whose pathname is more than 10 characters, the total pathname is more

than 64 characters and part of the directory name is truncated without

warning.  Then when the synthesis tools look for these files by name,

the files can't be found.  PC synthesis will either complain that the

xblox package can not be found, or if the truncation falls at just the

right spot, crash because it can read one of the files but not another.

Workaround:

-----------

Install prosynthesis into a directory

whose pathname is NOT more than 10 characters,

_______________________________________________________________________________

Bug ID:       18547               Reported:   Jan 13, 95

Runs out of memory when default cpu setting is medium on some circuits

Description:

------------

When attempting to synthesize some circuits during optimization all

real memory and swap are exhausted (~200Mbytes).  Viewsyn continues

mapping and creates wir file and report file after memory allocation

error occurs.  Here is an example error in the synth.log:

****Optimizing detect (partition 2)****

out of memory allocating 26873856 bytes

Workaround:

-----------

If running from the command line ensure synth.ini file has the line

cpu_effort low

If there is still a problem also try turning redundancy removal off

redundancy_removal off

If running from the gui set the menu pick settings with the values given above.

_______________________________________________________________________________

Bug ID:       18727               Reported:   Jan 23, 95

large buses cause viewsyn to crash before mapping.

Description:

------------

if a component is instantiated with a lot of I/O pins or a large I/O

bus, viewsyn will crash before mapping with the following error. This

bug is related to both the number of characters used in the bus name

and the number of bits in the bus

Workaround:

-----------

1) Ensure that bus_bit_width*busroot_name_length is less than 4096

2) Split busses up into smaller groups

3) Use short bus names.

_______________________________________________________________________________

Bug ID:       14921               Reported:   Jul 11, 94

Description:

------------

************************PC ONLY*

The entity name of a vhdl file requires users to conform to the 8 character

DOS limitation.  Problems may arise when exceeding that limitation, as

detailed in the chart below

Workaround:

-----------

Make sure that the entity name in your vhdl code is only eight characters

in length, and preferably only seven so that the screen looks right.  Also,

make sure that the file name matches the entity name, or the warning that

is generated will halt the operation of PROgen.

_______________________________________________________________________________

Bug ID:       16283               Reported:   Sep 15, 94

Synopsys style DFF inferencing does not produce optimum results for Xilinx

Description:

------------

The "Synopsys" style DFF inference for a DFF:

without async set/reset

with hold- (or load-) enable

does not result in an efficient circuit for Xilinx

e.g.

entity viewtech_fd1le_bad is

port (

D, CP, LE: in  VLBIT;

Q, QN              : out VLBIT);

end viewtech_fd1le_bad;

architecture behv of viewtech_fd1le_bad is

signal tmp: VLBIT;

begin

process(cp)

begin

if (cp'event and cp='1') then

if (le='1') then

tmp <= d;

end if;

end if;

end process;

q <= tmp;

qn <= not tmp;

end behv;

Workaround:

-----------

Use the "wait until" style:

entity viewtech_fd1le is

port (

D, CP, LE: in  VLBIT;

Q, QN              : out VLBIT);

end viewtech_fd1le;

architecture behv of viewtech_fd1le is

signal tmp: VLBIT;

begin

process

begin

wait until (cp'event and cp='1');

if (le='1') then

tmp <= d;

end if;

end process;

q <= tmp;

qn <= not tmp;

end behv;

_______________________________________________________________________________

Bug ID:       17296               Reported:   Oct 25, 94

GUIs of different viewsyn are affected by broadcasts of other vsyn runs

Description:

------------

When multiple instances of viewsyn are simultaneously active; the hac

broadcasts to the GUI (from a particular instance) affect the displays

of all viewsyn instances.

Workaround:

-----------

This does not happen if the tool is invoked from the powerview cockpit.

_______________________________________________________________________________

Bug ID:       17568               Reported:   Nov 09, 94

Vsyn doesn't always perform iopad insertion when it should

Description:

------------

The io pad insertion is not always invoked correctly.  With two

unrelated VHDL files, A and B, in the same directory, when using the

-iopad option for B, vsyn doesn't write the -iopad option into the

vdes_cmd file and iopad does not get run if A was syntheisized first.

Workaround:

-----------

do vsyn -cleanup before synthesizing _each file_

_______________________________________________________________________________

Bug ID:       17782               Reported:   Nov 23, 94

Name collision between component instance name and entity name

Description:

------------

When a VHDL component entity name is the same as an instance of the component,

an error message is generated.  For example, given this VHDL:

00273     EISAMSTR: EISAMSTR

00275         port map(BCLK => BCLK,

00276                  BUSREQ => BUSREQ,

00277                  CMD => CMD,

00278                  CROSS1K => CROSS1K);

This error is generated:

Line  273:     Error <2> eisamstr redeclared

Workaround:

-----------

Change the instance name; for example, replace with the line

00273     EISAMSTR1: EISAMSTR

_______________________________________________________________________________

Bug ID:       17782               Reported:   Nov 23, 94

Division by two is limited to integer operands

Description:

------------

The following VHDL:

variable offset, a:bit_vector(31 downto 0);

begin

if t='1' then

offset:=extendum(a,32)/2;

else

offset:=x"00000000";

end if;

end process;

generates a design with all inputs unconnected, yet removing the divide

by 2 generates a reasonable design.

This is because support for the division operator is limited to integers,

and the above example applies division to a bit_vector which is the type

returned by extendum.

Workaround:

-----------

1. Convert the bit vector produced by the extendum to an integer (for

example, using the function v1d2int).

2. The function "shiftrum" will perform the same function since shifting

right is equivalent to division by a power of two.

_______________________________________________________________________________

Bug ID:       17885               Reported:   Dec 01, 94

Message "node_function: function is not minimum-base" appears for one design

Description:

------------

When synthesizing a hierarchical design, the following message appears:

Warning: Internal node <name> in <name> has multiple drives!

node_function: function is not minimum-base

Workaround:

-----------

This message does not appear when this design is flattened.  The design

then synthesizes correctly.

_______________________________________________________________________________

Bug ID:       18528               Reported:   Jan 12, 95

"Out-of-Scope" components cause erroneous area reports.

Description:

------------

When synthesizing a hierarchical design which contains a leaf component that is

"Out-of-Scope", i.e., instantiating a library component explicitly, the report

file for all components in the hierarchy which contain this leaf cell is

incorrect.  As an example, if you have the following hierarchy:

TOP_LEVEL

|______COMPA

|______COMPB

|_______"Out-of-Scope" library component

and assume that TOP_LEVEL, COMPA, and COMPB all have logic within them,

the report file for COMPB will show the Area/Cell for the library

component as 0 which is correct since it is out-of-scope.  However, COMPA

will show the Area/Cell for COMPB as 0 also, even though COMPB has a total

area number greater than 0.  This propagates all the way to the top of the

hierarchy so that TOP_LEVEL shows the Area/Cell for COMPA as 0 which in

reality is a very large number.

Workaround:

-----------

Generate correct area report manually.

_______________________________________________________________________________

Bug ID:       18569               Reported:   Jan 15, 95

After running viewgen then synthesizing, errors appear in downstream tools

Description:

------------

When synthesis runs, it writes out one "sheet" of a wir file.  When viewgen

runs, it may write out several "sheets" of schematics.  If the design is then

synthesized, the extra sheets of schematic and/or wir file will be left

around and will cause trouble in the simulation netlister "vsm" or other

tools which read netlists.

Workaround:

-----------

Before synthesizing, delete all sheets of the design from the schematic

and wir file directories.

_______________________________________________________________________________

Bug ID:       18576               Reported:   Jan 16, 95

Optimization runs for a long time or runs out of memory

Description:

------------

For one design, ViewSyn runs overnight during the optimization stage.

For a second design, ViewSyn fails in the mapping stage and the synth.err

file indicates that it ran out of memory.

Workaround:

-----------

1. Reduce the CPU effort requested.  As more optimization is performed,

more memory and time are consumed.

- If the option cpu_effort is set to medium or high, turn it one setting

lower.

- If the option redundancy_removal is set to "on", turn it off.

2. More efficient synthesis is possible by adding hierarchy and breaking

large combinational blocks into more than one element; each can then

be optimized at the more expensive settings.

3. Set the optimization to "none".  This will consume a smaller amount of

memory, but the quality of optimization will clearly be worse.

4. As a last resort, execute "vhdldes" from the command line; this uses

a lower effort optimizer: "vhdldes -tec=<technology> <design>"

_______________________________________________________________________________

Bug ID:       18721               Reported:   Jan 23, 95

int2v1d in pack1164.vhd does not handle negative integers

Description:

------------

The simulation function to convert integers to one dimensional bit vectors

does not handle negative integers correctly.  While the circuit synthesizes

correctly, it will simulate incorrectly with a full VHDL simulator.

Workaround:

-----------

Negative integers require a special purpose function to handle the

sign bit extension correctly.

_______________________________________________________________________________

Bug ID:       18739               Reported:   Jan 23, 95

Message "error opening pinorder file" appears on large hierarchical designs

Description:

------------

When the synthesizer creates a large symbol to represent a hierarchical VHDL

design,  it must write the information about the symbol ports into a "pin"

file, that is, sym\<design>.pin.  This pin file is equivalent to placing

a PINORDER attribute on the symbol; however, for a PINORDER whose length

is over 128 characters, the pin file must be created instead.

When the pin file should be created, vhdldes instead reports:

Error opening Pinorder file....

<design> written

It creates the symbol correctly but the pin order information is incorrect.

Workaround:

-----------

1. run the utility "vhdl2sym" on hierarchical elements in a design to

create symbols first.  Vhdl2sym correctly creates the pinorder file.

2. enter the pinorder file manually, one pin name per line.

_______________________________________________________________________________

Bug ID:       18892               Reported:   Feb 01, 95

Errors from XBLOX appear after flattening one design

Description:

------------

Simple XBLOX designs go through with no errors.  However, this design

causes errors to be produced by the XBLOX tool.  The messages complain

about a missing ELEM field in the XNF file.

Workaround:

-----------

1. This occurs if the "primary directory" contains symbols named INELEM or

OUTELEM.  Previous versions of the synthesizer would create these XBLOX

symbols in the user's directory.  If the symbols are present, delete them.

2. For some designs, the flattening process seems to cause the ELEM attributes

to be left off.  Turning off flattening may fix the problem.

_______________________________________________________________________________

Bug ID:       18899               Reported:   Feb 01, 95

Message "vcore-E-1320 vcRegionData called on destroyed region" can be ignored

Description:

------------

On one hierarchical design, the design is run first flattening pre-opt.

When the design is recompiled and run again without flattening, several

instances of the message

vcore-E-1320 vcRegionData called on destroyed region

appear.  These messages may be ignored.

_______________________________________________________________________________

Bug ID:       19105               Reported:   Feb 15, 95

Toshiba libraries removed from release; now supported by Toshiba

Description:

------------

The Toshiba synthesis libraries(tc*.sml files) are not included in Vsyn

release tape anymore. This is because there is an agreement for Toshiba

to provide these files and not Viewlogic. You may find some old copies

of tc*.sml files from previously installed Viewlogic 52/53 standard

directories, which may be out-of-date. Please contact your local

representative or vendor for new libraries.

_______________________________________________________________________________

Bug ID:       19106               Reported:   Feb 15, 95

Description:

------------

There is no explict command provided in vsyn50 beta for excluding gates.

Workaround:

-----------

The workaround is to add "exclude [list of gates]" command to the

<entity>.mcm file under synth/<entity> directory. Note that, "setting

timing constraints" from the gui overwrites the .mcm file. Therefore,

the "exclude" line of command should be added into .mcm file after the

timing constraints is set.

_______________________________________________________________________________

Bug ID:       18001               Reported:   Dec 07, 94

Viewgen doesn't connect bundles properly

Description:

------------

The current version of viewgen for 5.3 does not always correctly

assemble bus bundles as busses for subsequent display in Viewdraw.

When a Synthesis generated schematic is viewed in ViewDraw

there may appear to be missing nets and other warning messages.

e.g. "Schematic file <> has invalid number of joints or segments on net"

The wir file generated prior to viewgen is correct.

However Viewgen may also generate an incorrect wir file which overwrites

the correct version. Please regenerate the WIR file from

synthesis and do not invoke the View Schematic command from synthesis

if you have this problem. This should ensure you simulate with the

correct WIR file.

III.  Bugs fixed in this release

4127 viewgen does nothing with his busses.

8496 Warning about mismatched pin_count is incorrect.

8676 Full VHDL requires pack1076 library: PACKAGE not found or must be

analyzed...

9426 core dump

10037 synth segmentation fault core dump

12627 vdes core dumps in 2 cases / working on Fix

12859 rom512: Design no longer runs when ViewArchitect is used.

13042 When referencing general system library packages, comments a

13046 Use VHDL Instance names in the WIR file

13395 wir file created is not correct

13901 core dump when "clean" or "clean_con" entered with no design

13912 real09 design runs out of memory during VHDL analysis

13938 Fannout buffering can be  incorrect for hierachical circuits

14492 Non-inverting buffers are not being chosen with default input loading

14578 wir command in VHDLDES doesn't write busses when symbols exist

14809 SML files are not read if there is UPPERCASE in the unix path

14965 vhdldes behaves  different when the order of vhdl files is c

15108 incorrect error message produced by vhdldes 3.0

15635 IOPAD passes only one attribute on a net

15684 If the out_dir alias is set, iopad fails.

15765 Constraints are not being accepted.

15896 SML output should be plain-text

15897 Critical Path info is printed on screen, but not in .rpt file

15909 Timing Anlaysis reports are not reasonable

15943 Hierarchy Control window should not be popped to the front each time

15945 Comments on Hierarchy Control window.

15950 View->Generate Schematic does not seem to work

16024 After compiling a design, Setup->View Design should be done automatically

16025 Compile->VHDL Source Files list is not alphabetized.

16027 Bus erro rwhile compiling multiple files

16029 Bus error compiling dff8.vhd

16031 changed relevant "error()" calls to "vlEx.Throw2()"

16044 line 55: Error <196> Constant expression expected

16047 Type mismatch error specifes reference to vlbit_1d

16408 VIEWFPGA does not terminate properly with -iopad arg in Crea

16132 Timing Report header needs to be cleaner

16172 ViewSynth Requires 2 Licenses During Mapping

16218 Typo in help - Set constraints.

16285 make facility not recognizing design was optimized

16465 synthesis blows up with/without map first on multiple OR condition

16555 vlpack: error when vdd is connected to gate input

16558 "An exception has occured" on my vhdl file

16570 Generics on entity cause 'Intermediate Node Has No Drive' Error

16715 Huge number of warnings generated by vhdldes on fpgasyn output

16718 File COMPONENT needs to go somewhere

16720 Xilinx iopad operation should not automatically be run on top level

16722 Hac does not remove hac.lck under some normal exit conditions

16776 Mapping Fails: module not found

16931 xblox design -> hac 3.0 -> Xilinx tools fails ; -> vhdldes 3.0 -> O.K.

16977 sml file has negative load.

17012 inputs to dffs are grounded

17027 Scrolling Diagram Corrupts graphics of other applications.

17028 Icons indicate MAPPED when they are only COMPILED

17075 Can not select a VHDL compiled design

17076 Zoom in function does not work correctly.

17096 Internal error (Exception in HAC) during vhdl compile and dissolve.

17117 Some instances failed to be dissolved (pre_map)

17119 Vs3.0 creates large level of gates

17176 clear work area: hac -clean does not remove top.ref

17194 iopad: Error 1403: Failed to find pin D on symbol XC4000:BUFT

17197 "Completed all operations, no errors" no matter how many errors vhdm finds

17198 vhdldes requires upper case aliases for technology symbols in viewdraw.ini

17199 Use "iw1level" instead of "iwgroup"; improved runtime and PC reliability

17202 Check to make sure technology is set before proceeding with _optimization_

17204 "An exception has occurred" in hac due to gnl syntax error

17210 10-6 ViewSyn30 build still consumes net licenses

17214 Giving hac a non existent enity to synthesize doesnt give warning

17221 Syntax error when running flatten pre-opt after compilering.

17227 Compile VHDL... pick list is not sorted

17250 Flatten does not dissolve all children

17282 hac failure: syntax error in generated gnl

17295 ViewArch created vhdl file cann't be compiled by viewsyn.

17389 Intermediate node has no drive message!

17409 Incorrect logic when using tri-states.

17471 Incorrect logic when using tri-states in hierarchy.

17526 Script error if you select viewdesign from setup menu.

17527 Show Schematic on top_level always brings up one of the lower_level

schematics.

17542 bad gnl generated

17543 bad gnl generated

17546 all test cases fail - differences between reference wir files and

generated ones

17547 fails on nets, sys3k, and sys4k

17567 Alucons should search for files along WDIR rather than needing an env.var

17576 iopad does not handle buses pins

17577 alucons run out of memory

17578 fpgasyn regression tests are not installed

17593 Technology selection has no entries.

17603 using variable produce bad logic.

17605 Internal error in hac while flattening GNL.

17606 Symbols created for designs with inout(bi)  pins are not correct

17622 Couldn't generate schematic for the design after running VLPACK

17623 Implement code review comments in RCS after Beta release

17646 "fpgasyn -xc4000" core dump

17664 Trouble with package selection!

17680 It always does optimization even when you set optimi off.

17706 Cannot get rid right mouse button menu..

17743 Cycle in network causes Fatal with Hierarchy, Works when Fla

17744 Hierarchy display doesn't take up the full window

17765 Mapping fails with bus error.

17820 ViewArchitect outputs cause ViwSyn segmentation violation

17845 Error in mapping.

17910 Mapping fails with bus error.

17986 Memory allocation error.

17989 Out of memory.

17991 Memory allocation error when synthesizing with large number of pins

18004 Inability to read VHDl file from any other directory then primary.

18021 Viewscript error crashes viewsyn when click on "Set Defaults" menu.

18028 Mapper runs "forever" on fp_sn_addcexp_comp_proc1.opt.

18051 Internal error: syntax error while xblox is instantiated.

18054 No more core memory left.

18080 Automatic IO pad insertion doesn't work correctly.

18141 Synthesis PACKS the design into CLBs irrespective of PACK on/off.

18156 core dump when analysis failed.

18163 core dump when analysis failed.

18164 Fails analysis if not supported in alucons

18165 View Design... should be grayed out

18191 hac core dumps when VL_MEMDBG is set.

18211 Segmentation fault during analysis.

18240 ViewSynthesis core dump during compilation.

18287 bad pack1164.vhd file.

18306 After compiling a new design the display shows a preexisting

18317 Error in mapping

18318 The synthesized circuit is broken!

18337 hac  reports "exception occured"  with "-fla_opt".

18347 core dump in v5.2 12/14/94, 0 length uartish2.lis file crea

18358 FIX FOR BUG 17566 REQUIRES CHANGES IN GUI..

18371 vhdldes core dump while generating top level symbol file

18372 viewFSM needs to communicate with ViewSynth GUI via ITC

18373 hac  reports "no error" even core dump occured.

18383 vsyn50 can't compile with Viewlogic standard platform.h

18387 Large negative rise and fall times in synthesis report file.

18412 Error in input port instantiation : a (a [0])

18465 GUI locks up after drawing hierarchy tree (with viewscript

18501 Directory selection is confusing

18510 Viewsyn is no longer performing load buffering

18549 Viewscript error when setting output constraints.

18554 vsyn fails to map fll.vhd from viewarchitect regression test

18565 vhdldes core dump while writing wir file for top level

18591 "Edit VHDL" does not owrk on design from ViewArchitect

18592 Incorrect synthesis of buses.

18618 iopad bus cleanup did not work for a very common case

18692 ALUCons Alert when technology not supported

18719 vhdldes core dump while synthesizing

18887 The ViewSyn Icon has a typo

18953 Incorrect synthesis of buses from shift register