*************************

**** Powerview 5.3.2 ****

**** Release Notes   ****

*************************

Introduction

Overview

Powerview is a high-performance UNIX-based electronic design

environment with native implementation of common GUIs. Powerview is a

suite of CAE tools within an environment that is based on the CFI (CAD

Framework Initiative) 1.0 specification.  Powerview supports your

design process through innovative tool technology, efficient project

management, and a configurable modular architecture.

These Release Notes outline the Powerview 5.3.2 features and

enhancements, as well as additional information regarding Powerview.

Platform Support

Powerview 5.3.2 platform support includes the following:

*  Sun 4/SPARCstation running X11, or OpenWindows, under SunOS V4.1.3

or V5.3 (Solaris 2.3 or 2.4)

*  IBM RISC System/6000 running X11 under AIX V3.2.5 and 4.1.1

*  Hewlett-Packard 9000 Series 700 running X11, or HP VUE, under

HP-UX V9.05

*  DEC Alpha AXP workstation running X11, or DECwindows, under Alpha

OSF/1 V3.0

NOTE: The last update to DECStation software was the Powerview 5.3 release.

Installation & Media Distribution

All 5.3.2 tools need to communicate with a 5.3.2 VNSD (that is, a

5.3.2 VNSD will try to terminate automatically a pre-5.3.2 VNSD on the

local host even if the older VNSD is owned by another user). Follow

the directions displayed by VNSD and/or the Powerview Installation

manual for more information.

5.3.2 VNSD supports 5.3.2 and pre-5.3.2 Powerview tools. A 5.3.2 VNSD

needs to be running on the license server machine so 5.3.2 tools can

check out licenses.

A 5.3.2 VNSD defaults to operating with most broadcasting messages

turned off.  However, full broadcasting can be turned on by the user.

Scenarios of migrating/co-existing to/with 5.3.2 VNSD are suggested in

the Powerview Installation manual.

Solaris 2.4 has fixed a socket problem that enabled 5.3.2 VNSD to use

the input port immediately after another 5.3.2 (or later) VNSD is

terminated. However, because previous Powerview releases used Solaris

2.3, the user will need to wait (approximately four minutes according

to SUN) before a 5.3.2 VNSD can use an input port released by a

pre-5.3.2 VNSD. The input port released by a pre-5.3.2 VNSD upon

exiting is held by Solaris for the previously mentioned waiting

period.

CD-ROM is the default* distribution media for all platforms.

Powerview products are installed from the CD to local disk. On-line

documentation (ViewDoc) is also the default publication set for all

platforms.  ViewDoc and the libraries are not directly accessible from

the CD. (*Tape distribution is available by request.)

Licensing Changes

Powerview 5.3.2 uses a license file with a combination of versions

4.10, 5.10, and 5.20 license groups (see the license.tmp in the

Powerview Installation manual).

NOTE: There is no VERSION 5.30 group.

New products added to the version 5.20 group include:

*  LMC Swift Interface - PRODUCT 581

*  ViewFloorplanner - PRODUCT 631 - 641

*  UniCAD PCB Interface - PRODUCT 648

*  ViewFlow II - PRODUCT 502

*  ViewDatabook II - PRODUCT 644

*  ViewLibrarian - PRODUCT 646

ViewDoc

The Powerview 5.3.2 release includes a new version of ViewDoc.  This

new version includes the following new features:

*  This release of ViewDoc introduces a new user interface.

Noticeable differences include:

*  The icon bar

*  The Page menu

*  The Browser => View command

*  The View => Back command

*  Solution collections

This release of ViewDoc organizes the Powerview documentation into

several smaller collections, referred to as solution collections.

Each solution collection corresponds to a Powerview software solution

package.  Solution collections consist of:

*  Powerview Base Documentation

*  Design Entry and Digital Simulation Solutions

*  PCB Solutions

*  Analog and Mixed A/D Solutions

*  ASIC/FPGA Solutions

*  Programmable Solutions

*  Team-based Engineering Solutions

*  Additional Powerview Documentation

*  Environment Integration Solutions

NOTE:   The Powerview CD-ROM does not include the Environment

Integration solution collection.  You will only receive this

collection if you are receiving a separate Environment

Integration product.

You can choose to install one or all of these collections.

IMPORTANT!

You must install the "Powerview Base Documentation" collection in

order to invoke ViewDoc.  This solution collection contains the

ViewDoc executable, as well as the basic set of Powerview manuals.

You can select to install additional solution collections during the

installation process.

*  Meta-collections

A meta-collection is a combination of two or more solution collections

combined into one new collection.  You can access and search

meta-collections just like any other collection.

For more information, refer to the ViewDoc User's Guide.

New Products

Powerview 5.3.2 includes the following new products:

*  Allegro Integration Package - A new user interface upgrade to the

existing Allegro interface for the addition of Allegro-centric

attributes. This integration package provides connections to the

ViewFloorplanner and Precise products.

For more information, refer to the Allegro Integration Package

User's Guide.

*  ViewFloorplanner - An interdisciplinary design automation tool for

engineers and designers that allows "what if" analysis early in the

design process.

For more information, refer to the ViewFloorplanner documents.

*  ViewFlow II - Provides the ability to build and run design flows.

Flows can be drawn graphically in edit mode, and the tools can be

executed and the results displayed in run mode. Data objects can

be edited and data managed.

ViewFlow II provides data management capabilities via the VDM

library; the full capabilities of ViewData are available through

ViewFlow II.

NOTE: See the following note on RCS encapsulation at the end of this

section.

For more information, refer to the ViewFlow II User's Guide.

*  ViewLibrarian - A command center for specifying component data

sources, entering and modifying component data, and verifying the

component data. Provides component data management functions for

various storage managers, and facilitates the library release process.

NOTE: See the following note on RCS encapsulation at the end of

this section.

For more information, refer to the ViewLibrarian User's Guide.

*  ViewDatabook II - Allows you to specify component search criteria,

execute the search, and display the search results. Enables viewing

any data associated with a component found in the search, and

instantiation of that component into a design entry tool, such as

ViewDraw.

NOTE: See the following note on RCS encapsulation at the end of

this section.

For more information, refer to the ViewDatabook II User's Guide.

IMPORTANT!

If you are using the RCS ViewDataBUS encapsulation (vdmRCS), before

running tools built on the 5.3.2 version of ViewDataBUS (such as,

ViewData, ViewDatabook II, ViewFlow II, and ViewLibrarian), you should

make sure that you do not have any data residing in the working area

of those locations that use the RCS encapsulation.

The 5.3.2 ViewDataBUS RCS encapsulation keeps track of, and requires

more information than, previous versions of the RCS encapsulation. It

obtains this information when an object is checked out. Therefore, in

order for the new version of the encapsulation to work properly, every

object that the encapsulation is to manage must undergo the checkout

operation.

Powerview Product Enhancements

ViewDraw

The ViewDraw product enhancements include:

New utility:

* sdborder - a stand-alone utility that orders the contents of

schematic and symbol files. It is used primarily with data management

systems, like RCS, at the time of file check in to reduce disk space

because RCS stores the deltas of file versions.

New commands:

* Select => Group - allows you to use an object filter to select only

objects of a certain type on the entire sheet or from those currently

selected objects (keyboard command: sgroup)

* Select => Owner - when an attribute or label is selected, ViewDraw

selects the owning object, for example, the object to which the

attribute or label is attached, whether it is a pin, net component, or

component pin (keyboard command: sowner)

* Change => Pin => Number => Swap - swaps the pin numbers of two

component pins (keyboard command: pinswap)

New mode:

* Full VHDL Checks - Toggles on or off to indicate whether additional

VHDL design rule checks will be made.

Refer to the ViewDraw Reference Manual and Schematic Design User's

Guide for more information.

ViewGraph

There is a new function called vgrOccBlockPushSheet.

ViewScript/ViewDraw API

The ViewScript/ViewDraw API product enhancements include:

New functions:

*  BlockFree - frees the memory associated with the given block

* DesignIterComps - iterates through the components in a design and

maintains the hierarchy

* NetIterCompPin - iterates through the component pins that are

graphically connected to a given net

Refer to the ViewScript/ViewDraw API manual or the man pages for more

information.

ViewData

The ViewData product enhancements include:

*  Creating and restoring checkpoints on a collection object

*  Updating the status of an individual object

*  Instantiating new data objects

*  Displaying objects only in the working area,  storage area or both

*  Before and after methods for storage management operations

*  Setting of symbolic tags on an object

*  Permanent attributes on a particular version of a data object

*  Ability to read in location definitions from .ini files

*  ViewData PI for programmable extensions

*  Class type and location browsers for easier information retrieval

IMPORTANT!

If you are using the RCS ViewDataBUS encapsulation (vdmRCS), before

running tools built on the 5.3.2 version of ViewDataBUS (such as,

ViewData, ViewDatabook II, ViewFlow, and ViewLibrarian), you should

make sure that you do not have any data residing in the working area

of those locations that use the RCS encapsulation.

The 5.3.2 ViewDataBUS RCS encapsulation keeps track of, and requires

more information than, previous versions of the RCS encapsulation. It

obtains this information when an object is checked out. Therefore, in

order for the new version of the encapsulation to work properly, every

object that the encapsulation is to manage must undergo the checkout

operation.

Refer to the ViewData User's Guide for more information.

ViewGen

The ViewGen product enhancements include:

*  Support for net bundles

*  Improved support for bus representation and mapping

*  Identifying potential error conditions of invalid object names

prior to the creation of schematics

*  Integration with ViewSyn 5.0.1

*  Creation of buses to map to the appropriate top-level symbols

*  Automatic creation of component names

*  Proper scoping of global nets

*  Automatic backup of schematics during the recreation of schematics

*  Enhanced message handling - detailed log file for easy error tracking

*  Improved error recovery and performance (speed)

PCB Interfaces V3.10

The UniCAD interface is a new, tightly integrated connection to the

UniCAD physical design environment. It provides forward and backward

connections, in addition to update capabilities, to the UniCAD

standard DD file.

ViewSim

NOTE: ViewSim is at release 5.3.1 in the Powerview 5.3.2 release.

The ViewSim product enhancements include:

*  Saber/ViewSim integration

For information on Saber/ViewSim integration see the Saber/ViewSim STI

User's Guide.

*  LMC Swift SmartModels interface support

For information on Swift SmartModels interface support See Appendix D

Using SmartModels in the ViewSim Reference Manual.

Note: If you currently use LAI SmartModels and have customized your

symbol library, both Viewlogic and LMC have tools to simplify the

migration from the old LAI interface to the new SWIFT interface. When

you choose to migrate contact your Viewlogic Application Engineer or

LMC for more information.

*  The second phase of strength handling that includes:

*  Displaying the circuit driver value

See the display => info command in Chapter 6 ViewSim Command

Reference in the ViewSim Reference Manual.

*  The reportx option to the defaults command warning you when a

signal is unstable or in an unknown state.  You can use this

command and option to flag bus contention.

See the Defaults Reportx command in  Chapter 6 ViewSim Command

Reference in the ViewSim Reference Manual.

*  The if command now works dynamically during simulation

See the If command in  Chapter 6 ViewSim Command Reference in the

ViewSim Reference Manual.

*  Reduced the time required to initialize data files associated with

large memories that use the ViewSim Builtin memory models.

*  Reduced the time required to read the .vsm file.

ViewSim/VHDL and ViewVHDL

NOTE: ViewSim/VHDL and ViewVHDL are at release 5.3.1 in the Powerview

5.3.2 release.

The ViewSim/VHDL and ViewVHDL product enhancements include:

*  The latest Vantage 5.2 analyzer and simulation kernel that gives

you substantial VHDL simulation performance improvements

*  VHDL to VHDL direct connections for VHDL nets, called Vantage_only

nets, in schematic based designs Vantage_only nets support all VHDL

data types and simulate much faster than other ViewDraw nets.

See Using Vantage_only Nets in Chapter 3 ViewSim/VHDL Analysis and

Simulation in the ViewSim/VHDL User's Guide.

*  Support for the Vantage STYX model interface that is a C interface library

See Appendix E Using The Vantage Styx C Model Interface Library in the

ViewSim/VHDL User's Guide.

*  Support for accessing elements of arrays and composites in the user

interface and debugging commands

See Accessing The Elements of Composite Types in Chapter 3

ViewSim/VHDL Analysis and Simulation in the ViewSim/VHDL User's Guide.

*  Different colors for keywords and comments in the source window

For information on the colors in the source window see The Source

Window in Chapter 5 The ViewSim Debugger in the ViewSim Reference

Manual.

*  Access to the Vantage CodePerfect tool to report on VHDL source

code line executions and VHDL signal or variable toggles

See Appendix G Using The Vantage CodePerfect Tool in the ViewSim/VHDL

User's Guide.

*  When you enter an interactive state using a break command, then all

simulators are synchronized at that time and ViewSim/VHDL executes all

commands waiting or applied through a command file are executed at

that time. (Previously the simulation would continue without looking

at the commands just read from the command file).

*  The VHDL_CONFIGURATION attribute is not required any more. However

if you attach a VHDL_CONFIGURATION attribute, ViewSim/VHDL uses it and

that VHDL_CONFIGURATION attribute will have higher precedence over the

VHDL attribute. If you don't attach VHDL_CONFIGURATION attribute, but

have attached only the VHDL attribute, ViewSim/VHDL first looks for a

configuration with that attribute value. If it finds one, it uses that

attribute to get the entity or architecture. If the configuration is

not found, ViewSim/VHDL assumes that the given name is an entity. Thus

you'll have one less attribute to worry about.

NOTE: If you have pre 5.3.1 ViewSim/VHDL you need VHDL_CONFIGURATION

and if both VHDL_CONFIGURATION and VHDL are present VHDL takes

precedence over VHDL_CONFIGURATION.

NOTE: The ViewDraw Level =>Push command could not be modified in time

for 5.3.1 release.  Hence in order for this to work, you need the

VHDL_CONFIGURATION attribute.  Also if both VHDL_CONFIGURATION and

VHDL are present VHDL takes precedence over VHDL_CONFIGURATION.

NOTE: If you don't want to get into this confusion, we recommend not

attaching both VHDL_CONFIGURATION and VHDL to the same instance or

symbol.

Viewsim/VHDL 5.3.1 comes with four versions of Vantage libraries. Some

of these versions are not compatible with previous library versions so

you must re-analyze your designs if you install one of these

versions. Some of these versions are optimized to increase simulation

performance. The contents, optimization, and re-analysis requirements

of these versions are in the following table.

Library Version

Library Contents and Optimization

Re-analysis Requirements

libs.04

Highly optimized Vantage and IEEE libraries

Must re-analyze

libs.dft

Regular libraries

Re-analysis not required

libs_syn.04

Highly optimized Vantage, IEEE, and Synopsys libraries

Must re-analyze

libs_syn.dft

Regular Vantage, IEEE, and Synopsys libraries

Must re-analyze

Install libs.04 or libs_syn.04 to increase simulation performance.

Install libs.dft to avoid re-analyzing your designs.

Verilnet (Verilog Netlister)

The Verilnet product enhancements include:

*  REFDES option - The -refdes option directs the netlister to use the

value of the REFDES attributes on components for their instance names,

instead of their label or UID.

*  SERIAL option - The -serial option makes Verilnet generate serial

dummy net names for unconnected component pins. If the optional

[basename] is specified, Verilnet uses this as the base string name of

the serial signals. The default [basename] is unconn. For example, the

net names unconn0, unconn1, etc. are created by default.

*  DEFPIN option - The -defpin option allows you to specify a default

pin TYPE.  Verilnet uses this if no PINTYPE attributes are found on

symbol pins in the design. The syntax of the option is -defpin[type],

where [type] can be one of the following: IN, OUT, BI, input, output,

or inout.

* Components of type 3 Hetero (split ICs) are now combined into a

single component.

* Buses of the form XX[m:n]YYY are translated to legal Verilog HDL by

converting them to the form XXXYYY[m:n].

AnalogVIEW

Updates to cross probing to use ViewDraw component and net names for

the AnalogVIEW stimulus file when using the HSPICE simulator. The

names are translated internally to HSPICE name syntax before launching

simulation. You can still use HSPICE component/net names in their

stimulus text window.

ViewArchitect

The ViewArchitect product enhancements include:

*  Improved ViewArchitect/ViewSyn integration - a viewdraw.ini file by

ViewArchitect whenever a ViewSyn netlist is exported after the

synthesize process in the directory where the ViewSyn netlist is

generated

*  Interrupt capability - interruption of ViewArchitect processes with Ctrl-c

*  Deletion of components with unconnected output ports -

ViewArchitect deletes any Vware components with unconnected output

ports that are created.

ViewFPGA

Updated to work with ViewSyn 5.0.1

ViewSyn V5.0.1

New GUI:

*  Hierarchy browser

*  Selection of multiple VHDL files for compilation through pick menu

*  Incremental work in progress state change information via icons

*  Component entity hierarchy dependency tree display

*  Constraint setting via the GUI on selected blocks

*  Links with Viewlogic ITC

Hierarchy controller:

*  Support for hierarchical designs

*  Hierarchical manipulation dissolve, flatten, clone (unique entities)

*  Hierarchical timing exported via SML library files per entity

*  Accessible from command line as Vsyn command

*  Selection of Multiple VHDL files for compilation

(Vsyn -vhdl *.vhd)

*  Intelligent make means that one can incrementally compile

hierarchical designs

*  Support for partitioned and hierarchical ViewArchitect VHDL

VHDL improvements:

*  Support of Std_Logic built in (first available in 2.4)

*  Better performance on ranges in large Case statements

*  Provision of a Std_logic VHDL simulation package for use with Vantage

*  Support for ViewSyn and ViewArchitect specific attributes

*  Support for partitioned designs from ViewArchitect

*  Increased Synopsys compatibility for sequential constructs

*  Less restrictive syntax to allow more use of clock enable

inferencing for Xilinx

*  Ignore configuration and generic statements

*  Correct code now generated for certain If-Else statements

Improved logic optimization:

*  New optimization engine gives improved results

*  Redundancy removal option to produce 100% testable logic

Improved capacity:

*  Hierarchy and improved logic optimization increase capacity from

5-10k to 100k +

*  Single entity capacity (without hierarchy) improved

Improved mapping capability:

*  Fanout compensation based on repowering and tree buffering

*  Hierarchical interblock buffering added

*  Hierarchical timing and area information exported and reported

Xilinx features:

*  Use of clock enable flip-flops to reduce CLBs used (first available

2.3.1, less restrictive syntax supported in this version)

*  Mapping to IO pads to use built in flip-flop (first available 2.3.1)

*  Xilinx specific first generation specific CLB mapping to give

accurate CLB utilization plus reports on CLBs and logic levels

*  Beta version of advanced CLB mapper II (no report available)

available from command line

Links to layout:

*  Path constraint information for Xilinx place and route passed to

XNF via the WIR file

Improved support for buses:

*  Automatic generation of Viewlogic symbols with bused pins

*  Generation of buses in WIR file for symbols with bused pins

*  ViewGen support for bundled buses

*  Schematic displayed with buses

Compatibility mode:

*  The old 2.x vhdldes command line is still supported (however, you

cannot access hierarchy features, improved logic optimization, and CLB

mapper II)

Improved constraint setting timing analysis:

*  Timing analysis accounts for loads set on outputs

Reporting:

*  Improved timing report with critical path information

*  Critical path annotation on schematic via WIR file

*  Improved name traceability between register input and output pins

and original HDL signal names

Some features available by using ViewArchitect ahead of ViewSyn:

*  Better coverage of Synopsys 3.x subset

*  Operator inferencing

*  Datapath component generation based on area/speed gives better

results and uses less CPU

*  Operator mapping to X-Blox components gives better results for

Xilinx designs

*  FSM recognition and optimization

*  Resource sharing (see ViewArchitect product documentation for full details)

Some features available by using ViewFSM ahead of ViewSyn:

*  State diagram entry

*  Redundant state and unreachable state removable

*  Variety of state encoding schemes

*  Best in class FSM optimization (see ViewFSM product documentation

for full details)

Synthesis libraries:

*  act1, act2, act3, altera, ami2un, attperf, cg21, cgaau, cmos5,

cmos6, dio, lsi100k, lsi10k, lca300k, scmos, vgt350, vgx700, vs700,

vsc350, vsc370, xc2000, xc3000, xc4000 (For other libraries, contact

your local representative or vendor.)

Refer to the ViewSyn User's Guide and the ViewSyn VHDL Reference

Manual for more information.

DIVA Forward

The DIVA Forward product enhancements include:

*  A new preference, divafwd.alias net_name;net_name net_name;net_name

..., in the ic.ini file for aliasing net names. This is only used to

change a net name to a global net. The first net_name is substituted

with the second net_name.  Separate the net pairings with a

semi-colon.

To invoke divafwd from the command line with this preference, use the

divafwd -alias "net_name;net_name net_name;net_name ..." command. If

you invoke divafwd from a shell command line, you must enclose the

set(s) of net name pairings in quotes.

*  Modifiable rundir in mkcdsenv

DIVA LVS Navigator

A new zoom preference, set from the menu or ic.ini file, zooms on a

probed object in either the ViewDraw schematic or the extracted

layout.

HLD Forward

A new preference, hldfwd.h [On/OFF], in the ic.ini file for creating a

hierarchical netlist. The default is OFF, causing the creation of a

flat netlist. To invoke hldfwd from the command line with this

preference on, use the hldfwd +h design command.

ViewText

A horizontal scrolling preference, vtx.hscroll, displays horizontal

scroll bars in any ViewText or ViewText-based tool. The default value

is FALSE. A value of TRUE displays the horizontal scroll bar.

Misc.

The workview.msg file has been moved back to the /standard

directory. In Powerview 5.3.1, the workview.msg file was put in the

/version subdirectory. Some tools were unable to locate it there.

Announcements

Distribution Change

The following products are not included on the Powerview 5.3.2 CD:

*  ViewTest

*  ViewTime

*  ViewVCS

These products are available on tape. The product .tef and .xbm files,

which enable cockpit encapsulation and invocation, are still available

on the CD.

Retired Products & Utilities

The following products are no longer shipped with Powerview 5.3.2:

*  ViewRetargeter

Gatran is an upgrade option.

*  ViewFSM

*  ViewSynthesis (on Sun4 only)

The replacement is ViewSyn.

*  CBDS PCB Interface

The replacement is the UniCAD PCB Interface.