SPRs for the Powerview 5.3.2 Release                       Active SPRs

______________________________________________________________________________

Facility:      COCKPIT

SPR Version:   5.3.1

Platform:      SPARC

SPR #:         18132

Description:                                                                 |

------------

When envoking powerview on a Solaris machine, note that the selected host

is set to None.  When envoking viewdraw or any other tool, note that the

selected host field in the Cockpit goes blank.  It should read the hostname

of the solaris machine that the tool is running on.

This field works as expected for all other platforms.

Workaround:

-----------

Solution:

---------

------------------------------------------------------------------------------

Facility:      EDIF2VL2

SPR Version:   5.3

Platform:      SPARC

SPR #:         16267

Description:                                                                 |

------------

Missing labels after translating an EDIF schematic file into Viewdraw via

edif2vl2.

Solution:

---------

The problem with missing labels is related to limitations in the EDIF

language. In EDIF unlike ViewDraw,labels are associated with NET and not net

segments. When a net consisting of several DISJOINT segments (contains more

than one label), the label association in EDIF is neither implied nor stated.

When these nets are read in and there is more than one contiguous group of

net segments, it is possible to misplace the labels. This problem is seen

mostly in systems in which labels are assigned to net segments and not nets

(Like VIEWDRAW).

Solution:

-------------

A solution is planned to resolve the connection (or lack thereof) for nets

and similar functionality is planned for busses.

------------------------------------------------------------------------------

Viewlogic Systems, Inc.             Page 1                      05/17/95

SPRs for the Powerview 5.3.2 Release                       Active SPRs

______________________________________________________________________________

Facility:      EDIF2VL2

SPR Version:   5.3

Platform:      SPARC

SPR #:         16270

Description:                                                                 |

------------

A bus is not appearing on the schematic after running edif2vl2.

The following warnings appear:

vdraw-W-2258:Schematic file <sch> has invalid segment.

Ignoring segment.  Maximum segments for joint read.

vdraw-W-2231:Schematic file <sch> has invalid label.

Ignoring label due to invalid object or sub object.

vdraw-W-2237:Schematic file <sch> has invalid number of joints or

segments on net.

Deleting corrupt net with ID x.

Workaround:

-----------

Avoid testing EDIF schematic translations by using "round trips" of the

old EDIF schematic writer and the new EDIF schematic reader.

Solution:

---------

This problem only occurs if an older version of vl2edif2 (pre-5.3) was used

to generate the EDIF schematic and is read by the Powerview 5.3 version of

edif2vl2.  vl2edif2 did not create valid bus rippers which can be read in

correctly.

------------------------------------------------------------------------------

Facility:      FPGA_XILINX

SPR Version:   5.3

Platform:      ALL

SPR #:         16110

Description:                                                                 |

------------

When XNF2WIR is run, it does not use the "library alias" information in the

viewdraw.ini file.  It picks up the first component in the viewdraw search

path which matches the name it is looking for, rather than picking it from

the correct library.

For example,  if the xc4000 is before the xc3000 library in the search

order and XNF2WIR is run, it picks off the flip-flops from the XC4000

library and complains about the signal GR that is used in the XC3000 library.

If XC3000 is the first library in the search order, XNF2WIR works.

Workaround:

-----------

When doing an xc3000 design, make sure that the xc3000 library is first

in the viewdraw.ini search path; when doing xc4000 design, make sure the

xc4000 library is first, etc.

Solution:

---------

------------------------------------------------------------------------------

Viewlogic Systems, Inc.             Page 2                      05/17/95

SPRs for the Powerview 5.3.2 Release                       Active SPRs

______________________________________________________________________________

Facility:      PCB_ALL

SPR Version:   5.3

Platform:      ALL

SPR #:         16070

Description:                                                                 |

------------

If a design is named "pcb" and then pcbfwd is run,  a pcb.err file is not

created and no warning messages are generated.

Workaround:

-----------

Pcbfwd is not producing an pcb.err file because if there are any errors with

check then a <design name>.err file is created.  In this case it is also

pcb.err.  Check takes ownership of the <design name>.err so pcbfwd will not

overwrite this.

Solution:

---------

Name your design something other than pcb.

------------------------------------------------------------------------------

Facility:      PCB_PADS

SPR Version:   5.3

Platform:      SPARC

SPR #:         16165

Description:                                                                 |

------------

When running pcbbck from a PADS design with the WriteBaf keyword set to yes

in the configuration file, the following error is generated:

PCB: Error 6367: file <file>.baf, line #: baf Parse error at column # #

Workaround:

-----------

This error occurs when new nets are introduced while the design is in PADS.

Upon running pcbbck with WriteBaf Yes, these new nets are entered in the

resulting .baf file as ???, which then causes the above error 6367.

Solution:

---------

1.  The proper way to back annotate nets is to add the net in PADS and rename

the net. Otherwise if you do not rename the net then the default signal

name is not a valid Viewdraw net name and thus not supported.

2. The recommended solution is to drive any netlist changes to your design

from the schematic.  Back annotate the optimized board information to

Viewdraw, make the netlist change(s) in Viewdraw, and forward annotate

the changes to PADS (ECO).

------------------------------------------------------------------------------

Viewlogic Systems, Inc.             Page 3                      05/17/95

SPRs for the Powerview 5.3.2 Release                       Active SPRs

______________________________________________________________________________

Facility:      PCB_RINF

SPR Version:   5.3.1

Platform:      ALL

SPR #:         18055

Description:                                                                 |

------------

There are attributes defined in the Attribute Pass List that does not allow

RINF to find the appropiate library components from the Recal Central Library

Database.

Solution:

---------

An updated rinf.cfg file will be available on the BBS in the Powerview Sun

4.1.x area

that can be downloaded for all platforms.

Otherwise the attribute pass list should be:

BeginAttPassList

Specific

|BeginOatAtts

COM REFDES

COM $OBJNAME

COM RINF_ID

NET  RINF_ID

PKG ROTATION

PIN #

COM DEVICE

NET $OBJNAME

|EndOatAtts

PPN SCHED

PIN PINTYPE

COM PINSWAP

COM NC

COM SIGNAL

PKG CLASS

COM HETERO

|PKG PKG_TYPE

PKG TOLERANCE

|PKG P/D_NUM

General

NET CHAR_Z

NET ACTUAL_Z

NET NET_MAX_DELAY

NET NET_DEL_ERROR

NET ACT_NET_DELAY

NET NET_MAX_STUB

NET STUB_ERROR

NET NET_MAX_XTALK

NET XTALK_ERROR

NET NET_MAX_LOSS

NET ACTUAL_LOSS

NET NET_LOSS_ERROR

NET NET_COST

NET NET_CLASS

NET SPACING_CLASS

NET ACTUAL_C

NET NET_BUS

NET NET_SHIELD

Viewlogic Systems, Inc.             Page 4                      05/17/95

SPRs for the Powerview 5.3.2 Release                       Active SPRs

______________________________________________________________________________

NET C_GROUND

NET FREQUENCY

PPN PIN_MIN_DELAY

PPN PIN_MAX_DELAY

------------------------------------------------------------------------------

Facility:      VDMF

SPR Version:   5.3.2

Platform:      ALL_UNIX

SPR #:         19577

Description:                                                                 |

------------

If a user has a collection object with versions 1.1 and 1.2, when the 1.1

version is checked out, the correct collection and metadata are checked out.

But, the collection members (and other metadata) are not zeroed out and

restored from the newly checked out metadata.

Solution:

---------

When using the VDM 1.x metadata policy (that is, the vdm.new_meta preference

is off, the default for Powerview 5.3.2) the metadata only be "restored and

zeroed" for the particular version in question, and the vdmVerLATEST version.

This is a limitation of the VDM 1.0 metadata architecture.  Because of this

limitation, users should not attempt to modify the metadata of versions of

an object which are not checked out.

------------------------------------------------------------------------------

Facility:      VIEWARCHITECT

SPR Version:   5.3.1

Platform:      SPARC

SPR #:         20222

Description:                                                                 |

------------

I have installed Viewsyn 5.0 and ViewArchitect.  When I click on the

ViewArchitect icon, I get the following strange information:

Allegro CL 4.1 [SPARC:R1] (5/29/94 10:57)

...Franz, Inc., Berkely, CA, USE...

Silcsyn license error "cannot find license file" for the silcsyn option

Could you please tell me what's wrong?  And what has

Allegro and a certain Franz Inc., to do with ViewArchitect, by

the way?

Solution:

---------

Allegro is CLSI compiler, not to be confused with the Allegro PCB product.

Franz is the developer of this product, used by ViewArchitect.  The error

from ViewArchitect is usually due to the existance of an environment

variable, called LM_LICENSE_FILE, which points the Silc executables to

a Flex LM server, not Viewlogic licensing.  In fact, the products now

do use Viewlogic licensing.  The customer in this case does not have

this env var set, sent info to dev.

From Martin L. Resnick:

There are a few things happening here.  When you click the icon, a

command of 'viewarchitect" is issued (according to the viewarch.tef file).

If the user only has Viewlogics ".../wv/sun4" directory in their PATH, then

Viewlogic Systems, Inc.             Page 5                      05/17/95

SPRs for the Powerview 5.3.2 Release                       Active SPRs

______________________________________________________________________________

they will get something like :

viewarchitect: Command not found.

What seems to be happening here is that the user has

"../wv/sun4/viewarch/exe"in

their path, which is wrong.  They should have ".../wv/sun4/viewarch/com" in

their

path.

From Koen:

It's obvious if you read the comment from the developer above that something

is wrong with the installation procedure of ViewArchitect.  If the developer

confirms that by default, you will get "command not found", that is really bad.

I finally got it to work, but it is not straightforward, and it is also

not documented.  This is what I had to do:

1.  set path = (.../pv531/viewarch/com ...)

2.  setenv WDIR (.../pv531/viewarch/standard ...)

3.  setenv EXEDIR .../pv531/viewarch/exe/

4.  setenv work dlswork

5.  Create a local dir, e.g. /home/koen/va

6.  setenv dlswork /home/koen/va

Comments:

1.  The CD should install viewarchitect in the Powerview directory.

I asked it to install in /pv531, not /pv531/viearch/com.

2.  WDIR directory should be .../pv531/standard, and not

../pv531/viearch/standard, which is not even the subdirectory

where viewarchitect can be found (=com subdir).  We always have

------------------------------------------------------------------------------

Facility:      VIEWDATABOOKII

SPR Version:   5.3.2

Platform:      ALL_UNIX

SPR #:         20346

Description:                                                                 |

------------

ViewDatabookII reports an error when it encounters a .par file with

components which share a common PART_NUMBER attribute.  There are cases

where this could happen legally in the design process.  For  example, a part

is in development, but hasn't been assigned a part number yet so the

value "?" is used.  A different part is also under development and gets "?"

for its part number.  This results in an error when ViewDatabookII sees two

different lines with the same PART_NUMBER.

Solution:

---------

In this case, PART_NUMBER is not a unique identifier (ParKeyField) for the

par files.  If there is a unique identifier in the par files, set the

vllm.ParKeyField to the name of that unique identifier.  If there is no

unique identifier in the par files, set the vllm.ParKeyField to "", and a

unique identifier will be generated for.

------------------------------------------------------------------------------

Viewlogic Systems, Inc.             Page 6                      05/17/95

SPRs for the Powerview 5.3.2 Release                       Active SPRs

______________________________________________________________________________

Facility:      VIEWFPGA

SPR Version:   5.3

Platform:      ALL

SPR #:         14979

Description:                                                                 |

------------

When a user re-executes the Create=>FPGA Wire file menu pick with the Create

schematic option selected, if the previous schematic is displayed in ViewDraw

the following error will be generated in the ViewFPGA GUI while it's trying

to run ViewGen:

unable to create backup copy

could not create backup for:vfpclassDARABASE:workview/vfpclassPROJECT:

/vfpClassLIBRARY.......

error init schematic sheet name:

This error is generated because a lock file exists for that schematic and

ViewGen can't overwrite it.

Workaround:

-----------

There are two ways to solve this problem:

(1) Close the Viewdraw schematic in question and continue.

(2) Delete the lock file that exists for the schematic in the LOCK

subdirectory under the SCH directory for the project.

------------------------------------------------------------------------------

Facility:      VIEWFPGA

SPR Version:   5.3.2

Platform:      DEC/ALPHA

SPR #:         20473

Description:                                                                 |

------------

ViewSynthesis is not shipped with Powerview 5.3.2 because is does not run

under OSF/1 V3.0 on DEC/Alpha.  Since ViewSynthesis and the synthesis

libraries are not shipped, synthesized wire files cannot be created for

FPGA designs.

------------------------------------------------------------------------------

Viewlogic Systems, Inc.             Page 7                      05/17/95

SPRs for the Powerview 5.3.2 Release                       Active SPRs

______________________________________________________________________________

Facility:      VIEWGEN

SPR Version:   5.3.2

Platform:      ALL_UNIX

SPR #:         20493

Description:                                                                 |

------------

Bus bundles may be optimized out of the schematics by Viewgen.  For example,

a wire  file contains a bus bundle with a name like SIGA,SIGB,SIGC,BUS[12:0].

This bundle connects to one component as SIGA,SIGB,SIGC,BUS[12:0] and is

connected to another component as only BUS[12:0].  Since this bundle is

not fully connected, it will be optimized out by Viewgen and the components

will be unconnected.  The following warning is issued by Viewgen:

[W] No segments attached to the bus:  SIGA,SIGB,SIGC,BUS[12:0]

Workaround:

-----------

Only the schematics are affected.  If no schematics are generated, Vsm can

be run on the original wire files and the design will simulate correctly.

------------------------------------------------------------------------------

Facility:      VIEWNAVIGATOR

SPR Version:   5.3

Platform:      ALL

SPR #:         15918

Description:                                                                 |

------------

The "object" navigation keyword is supposed to look for REFDES, Labels, and

UIDs matching the specific partid.  It works fine on both labels and UIDs but

fails to locate parts when given a REFDES.

------------------------------------------------------------------------------

Facility:      VIEWSIM

SPR Version:   5.3

Platform:      HP

SPR #:         16350

Description:                                                                 |

------------

Previously declared vectors are not deleted after a Restart command. If a

vector has been declared earlier in a ViewSim simulation, and then a RESTART

command is executed, the earlier vector is still retained in the ViewSim

database.

Workaround:

-----------

A NETWORK command will delete the vector from memory. However, this may

cause several different kinds of conflicts, if the same vector name is used

a second time, or if the arguments of the vector are declared differently.

Solution:

---------

------------------------------------------------------------------------------

Viewlogic Systems, Inc.             Page 8                      05/17/95

SPRs for the Powerview 5.3.2 Release                       Active SPRs

______________________________________________________________________________

Facility:      VIEWSIM/VHDL

SPR Version:   5.3

Platform:      SPARC

SPR #:         16098

Description:                                                                 |

------------

If the user does not have write permissions or does not own some pre-existing

files in the vsession subdirectory of the project, or the directory itself,

then Viewsim/VHDL will generate a cryptic error-message as follows:

**Error: Could not open the file

No permissions.

There is normally no indication of which file caused the failure. Often this

error occurs after the compilation of the top-level VHDL completed

successfully with a compilation_results written to the vsession directory.

Workaround:

-----------

Make sure you have permissions to the vsession directory and all the files

currently in it. If there is no vsession directory, make sure that you have

write permission to be able to create the directory in your project

directory.

Solution:

---------

------------------------------------------------------------------------------

Facility:      VIEWSIM/VHDL

SPR Version:   5.3

Platform:      ALL

SPR #:         16364

Description:                                                                 |

------------

When attempting to use any of the clock, wfm or assign commands on a VHDL

port signal which is bi-directional (PINTYPE=BI), it is noticed that the

command changes the value of the VHDL-port itself (by doing a

display /instance_label/VHDL_port-name), but not that of the net attached

to the port.

Workaround:

-----------

The Vantage simulator creates two internal signals to represent the

bi-directional VHDL port - an "in" signal and an "out" signal.

Viewsim/VHDL is currently unable to resolve the two signal-values into a

single-value representing the net on the Viewsim side. To achieve this, you

must append an "_out" to the name of the signal you use in the

above-mentioned commands. For example; if your VHDL-port is called

/my_VHDL_component/Qout, then to perform an assign of the port to a '1',

you must do the following:

assign /my_VHDL_component/Qout_out 1

Solution:

---------

------------------------------------------------------------------------------

Viewlogic Systems, Inc.             Page 9                      05/17/95

SPRs for the Powerview 5.3.2 Release                       Active SPRs

______________________________________________________________________________

Facility:      VIEWSIM/VHDL

SPR Version:   5.3

Platform:      ALL

SPR #:         17025

Description:                                                                 |

------------

You will encounter this problem only if you invoke the ViewVHDL Design

Browser from the Net button inside Viewsim/VHDL, and attempt to change the

directory to something other than the current project directory or the

directory you invoked Viewsim from. Although Viewsim will be able to load a

.vsm file from the new directory, another program spawned by Viewsim,

vifext, will look for the vsslib.ini file only in the current project

directory or the directory you invoked Viewsim from. You may get errors about

a missing vsslib.ini file, missing entities, or other vsslib.ini file

related errors.

IF you use the Net button in Viewsim to invoke the Design Browser, then

change the directory name from which you are getting the .vsm file, Viewsim

will reference and load the file you specified, but vifext will not look for

the vsslib.ini file in the new directory, but instead continue to look in

the directory you invoked Viewsim on.

Workaround:

-----------

Invoke the Design Browser from the ViewVHDL icon in the Powerview Cockpit,

or don't change the directory path from within Viewsim.

Solution:

---------

------------------------------------------------------------------------------

Facility:      VIEWSPICE

SPR Version:   5.3

Platform:      SPARC

SPR #:         16299

Description:                                                                 |

------------

Pole-zero analysis does not produce any results in design.out file

The .PZ (pole zero) analysis option should report the poles and or zeros in

the pole zero section of the .out file.  This section shows up in the .out

file but data is not reported.

Solution:

---------

This is a limitation in this release of ViewSpice.

------------------------------------------------------------------------------

Viewlogic Systems, Inc.             Page 10                     05/17/95

SPRs for the Powerview 5.3.2 Release                       Active SPRs

______________________________________________________________________________

Facility:      VIEWSYNTHESIS

SPR Version:   5.3.2

Platform:      DEC/ALPHA

SPR #:         20551

Description:                                                                 |

------------

ViewSynthesis V2.4.1 does not run under OSF/1 V3.0 on DEC/Alpha, and is

no longer available or shipped with Powerview V5.3.2.

------------------------------------------------------------------------------

Facility:      VIEWTIME

SPR Version:   5.2.1

Platform:      HP

SPR #:         15257

Description:                                                                 |

------------

ViewTime and/or the Motive Engine appear to hang during startup or restart on

the HP and DEC Alpha platforms.  The Motive version banner is printed but

doesn't respond afterwards.

Workaround:

-----------

This is caused by a bug in the TCSH shell and has been detected on HP and

ALPHA. This may also affect other Powerview applications on DEC Alpha  and

HP platforms.

Solution:

---------

Disable the use of TCSH when using ViewLogic Applications.  Use CSH or

some other command shell to invoke Powerview Applications.

------------------------------------------------------------------------------

Viewlogic Systems, Inc.             Page 11                     05/17/95