Study on Maglev Windmill

1. Aishwary R. kumbhalkar    KDKCE/EE   0968957206aishwarykumbhalkar470@gmail.com

2. Nilesh A. Vaidya                 KDKCE/EE   07798326574     nileshvaidya440@gmail.com

3. Pratik S. Tiwari                  KDKCE/EE   08149974758      pratik41@gmail.com

4. Zalendra D. Bhagat            KDKCE/EE   08407947696     zalendrabhagat001@gmail.com

Abstract – -  Magnetic levitation, maglev, or magnetic suspension is a method by which an object is suspended with no support other than magnetic fields. Magnetic pressure is used to counteract the effects of the gravitational and any other accelerations. The principal advantage of a maglev windmill from a conventional one is, as the rotor is floating in the air due to levitation, mechanical friction is totally eliminated. That makes the rotation possible in very low wind speeds. Maglev wind turbines have several advantages over conventional wind turbines

Keywords: Wind Energy, Magnetic Levitation, Power Generation, Magnets.

- INTRODUCTION

Wind is a form of natural energy. It is a natural power source that can be economically used to generate electricity. The way in which wind is created is from the atmosphere of the sun causing areas of uneven heating. In conjunction with the uneven heating of the sun, rotation of the earth and the rockiness of the earth’s surface winds are formed.

The terms wind energy or wind power describes the process by which the wind is used to generate mechanical power or electricity. Wind turbines convert the kinetic energy in the wind into mechanical power. This mechanical power can be used for specific tasks (such as grinding grain or pumping water) or a generator can convert this mechanical power into electricity. The wind turbine is used for conversion of kinetic energy of wind into electrical energy. The wind turns the blades, which spin a shaft, which connects to a generator and makes electricity.

The Maglev wind turbine design is a vast departure from conventional propeller designs. Its main advantages are that it uses frictionless bearings and a magnetic levitation design and it does not need to vast spaces required by more conventional wind tur-bines. It also requires little if any maintenance.

Magnetic levitation is supposedly an extremely efficient system for wind energy. The vertically oriented blades of the wind turbine are suspended in the air re-placing any need for ball bearings

- Need for Wind Power Technology

Renewable energy is generally electricity supplied from sources, such as wind power, solar power, geothermal energy, hydropower and various

forms of biomass. These sources have been coined renewable due to their continuous replenishment and availability for use over and over again.                      The popularity of renewable energy has experienced a significant upsurge in recent times due to the exhaustion of conventional power generation methods and increasing realization of its adverse effects on the environment.

Maglev wind turbines have several advantages over conventional wind turbines. For instance, they’re able to use winds with starting speeds as low as 1.5 meters per second (m/s). Also, they could operate in winds exceeding 40 m/s. It would also increase generation capacity by 20% over conventional wind turbines and decrease operational costs by 50%. This makes the efficiency of the system higher than conventional wind turbine. Currently, the largest conventional wind turbines in the world produce only five megawatts of power.

The turbine uses permanent type of rare earth magnets (neodymium) instead of electro-magnets and therefore it doesn’t require electricity to run. The friction between the turbine blades and the base can maximum power output.

- WIND POWER & WIND TURBINES

- Wind Power Technology

Wind power technology is the various infrastructure and process that promote the harnessing of wind generation for mechanical power and electricity. This basically entails the wind and characteristics related to its strength and direction, as well as the functioning of both internal and external components of a wind turbine with respect to wind behavior. Wind power, as an alternative to fossil fuels, is plentiful, renewable, widely distributed, clean, produces no greenhouse gas emissions during operation and uses little land.

- The Power of Wind

As mentioned earlier the effective functioning of a wind turbine is dictated by the wind availability in an area and if the amount of power it has is sufficient enough to keep the blades in constant rotation. The wind power increases as a function of the cube of the velocity of the wind and this power is calculable with respect to the area in which the wind is present as well as the wind         velocity. When wind is blowing the energy available is kinetic due to the motion of the wind so the power of the wind is related to the kinetic energy.

- Types of Wind Turbines

Many types of turbines exist today and their designs are usually inclined towards one of the two categories: horizontal-axis wind turbines (HAWTs) and vertical-axis wind turbines (VAWTs). As the name pertains, each turbine is distinguished by the orientation of their rotor shafts. The former is the more conventional and common type everyone has come to know, while the latter due to its seldom usage and exploitation, is quiet unpopular. The HAWTs usually consist of two or three propeller-like blades attached to a horizontal and mounted on bearings the top of a support tower. When the wind blows, the blades of the turbine are set in motion which drives a generator that produces AC electricity. For optimal efficiency, these horizontal turbines are usually made to point into the wind with the aid of a sensor and a servomotor or a wind vane for smaller wind turbine applications. With the vertical axis wind turbines, the concept behind their operation is similar to that of the horizontal designs.

The major difference is the orientation of the rotors and generator, which are all vertically arranged, and usually on a shaft for support and stability. This also results in a different response of the turbine blades to the wind in relation to that of the horizontal configurations. Their design makes it possible for them to utilize the wind power from every direction unlike the HAWTs that depend on lift forces from the wind similar to the lift off concept of an airplane. Vertical axis wind turbines are further subdivided into two major types namely the Darrieus model and the Savona’s model. Darrieus Model which was named after designer and French aeronautical engineer, Georges Darrieus. This form of this design is best de-scribed as an eggbeater with the blades, two or three of them bent into a c-shape on the shaft. Finnish engineer Sigurd Savo-nius invented the Savonius model. The functioning of this model is dependent on drag forces from the wind. This drag force pro-duced is a differential of the wind hitting by the inner part of the scoops and the wind blowing against the back of the scoops. Like the Darrieus model, the Savonius turbines will work with winds approaching in any direction and also work well with lower wind speeds due to their very low clearance off the ground.

Fig 2.3.1 - Vertical Axis Turbine

- Major Components & Operation of Wind Turbine

A wind turbine basically draws the kinetic energy from the wind and converts this power to electrical energy by means of a generator. Its operability is dependent on key components of the turbine and its response to the wind based on how it is built. Figure 2.5 shows an illustration of a conventional wind turbine and its parts. With this turbine, the blades receive the wind and are caused to lift and rotate. Depending on the wind speed the controller will start up or shut off the turbine. If wind speeds are right between 8 to 16 miles per hour, the turbine would start to operate but will shut down if speeds exceed about 55 miles per hour. This is a preventative measure because at very high winds the turbine could be damaged. The anemometer on the turbine calculates this wind speed and sends the information to the controller. The VAWTs usually do not have anemometers because they are usually used for low speed and small scale applications. The high speed shaft drives the generator to produce electricity and they are connected to the low speed shaft by gears to increase their rotational speed during operation.

Most generators usually require a rotational speed of about 1000 to 1800 rotations per minute so the gears increases them significantly from 30 to 60 rotations per minute to the electricity producing threshold. All these components sit on a tower usually made out of steel or concrete. The height of the tower is dependent on the size of the rotors and the de-sired amount of electricity generation. Taller towers serve as an advantage because wind speed is abundant with height so the rotors will work well with increased tower height and promote more and efficient electricity generation.

- THE WINDMILL

This section introduces and provides a brief description of the major components and factors that will contribute to an efficiently functioning wind turbine. These factors are wind power, the generator, magnetic levitation. Later sections will provide an in-depth look into the essence of each factor and its function and importance to the overall operation of the vertical axis wind turbine.

- Wind Power

Undoubtedly, the project’s ability to function is solely dependent on the power of wind and its availability. Wind is known to be another form of solar energy because it comes about as a result of uneven heating of the atmosphere by the sun coupled with the abstract topography of the earth’s surface. With wind turbines, two categories of winds are relevant to their applications, namely local winds and planetary winds. The latter is the most dominant and it is usually a major factor in deciding sites for very effective wind turbines especially with the horizon-tal axis types. These winds are usually found along shore lines, mountain tops, valleys and open plains. The former is the type you will find in regular environments like the city or rural areas, basically where settlements are present. This type of wind is not conducive for effective power generation; it only has a lot of worth when it accompanies moving planetary winds. In later chapters, more focus will be placed on the power of wind and effective ways to design wind turbines for optimal wind power production.

- Generator

The basic understanding of a generator is that it converts mechanical energy to electrical energy. Generators are utilized extensively in various applications and for the most part have similarities that exist between these applications. However the few differences present is what really distinguishes a system operating on an motors. With the axial flux generator design, its operability is based on permanent magnet alternators where the concept of magnets and magnetic fields are the dominant factors in this form of generator functioning. These generators have air gap surface perpendicular to the rotating axis and the air gap generates magnetic fluxes parallel to the axis. In further chapters we will take a detailed look into their basic operation and the configuration of our design.

- Magnet Arrangement

The magnet arrangement is also known as maglev, this phenomenon operates on the repulsion characteristics of permanent magnets. This technology has been predominantly utilized in the rail industry in the Far East to provide very fast and reliable transportation on maglev trains and with on-going research its popularity is increasingly attaining new heights. Using a pair of permanent magnets like neodymium magnets and substantial support magnetic levitation can easily be experienced. By placing these two magnets on top of each other with like polarities facing each other, the magnetic repulsion will be strong enough to keep both magnets at a distance away from each other. The force created as a result of this repulsion can be used for suspension purposes and is strong enough to balance the weight of an object depending on the threshold of the magnets. In this project, we expect to implement this technology form the purpose of achieving vertical orientation with our rotors as well as the axial flux generator

- Power Generation

When designing a generator it is important to have a firm grasp of the basic laws that govern its performance. In order to induce a voltage in a wire a nearby changing magnetic field must exist. The voltage induced not only depends on the magnitude of the field density but also on the coil area. The relationship between the area and field density is known as flux (Φ). The way in which this flux varies in time depends on the generator design. The axial flux generator uses the changing magnetic flux to pro-duce a voltage. The voltage produced by each coil can be calculated using Faraday's law of induction,

V= -N(dO/dt)

- MAGNETIC LEVITATION

In selecting the vertical axis concept for the wind turbine that is implemented as the power generation portion of this project, certain uniqueness corresponded to it that did not pertain to the other wind turbine designs. The characteristic that set this wind generator apart from the others is that it is fully supported and rotates about a vertical axis. This axis is vertically oriented through the center of the wind sails, which allows for a different type of rotational support rather than the conventional ball bearing system found in horizontal wind turbines. This support is called maglev, which is based on magnetic levitation. Maglev offers a near frictionless substitute for ball bearings with little to no maintenance. The four different classes are Alnico, Ceramic, Samarium Cobalt and Neodymium Iron Boron also known Nd- Fe-B. Nd-Fe-B is the most recent addition to this commercial list of materials and at room temperature exhibits the highest properties of all of the magnetic materials. It can be seen in the B-H graph shown in Figure 5.1 that Nd-Fe-B has a very attractive magnetic characteristic, which offers high flux density operation and the ability to resist demagnetization. This attribute will be very important because the load that will be levitated will be heavy and rotating high speeds, which will exhibit a large downward force on the axis. The next factor that needs to be considered is the shape and size of the magnet which is directly related to the placement of the magnets. It seems that levitation would be most effective directly on the central axis line where, under an evenly distributed load, the wind turbine center of mass will be found as seen in Figure This figure shows a basic rendition of how the maglev will be integrated into the design.

Fig 4.1 - Magnetic Levitation

- CONCLUSIONS

At the end of the project, the magnetically levitated vertical axis wind turbine was a success. The rotors that were designed harnessed enough air to rotate at low and high wind speeds while keeping the centre of mass closer to the base yielding stability. The wind turbine rotor levitated properly using permanent magnets, which allowed for a smooth rotation with negligible friction.

REFRENCES

[1]. MagLev Wind Turbine Technologies, Inc. (MWTT) & Off Grid Technologies, Inc. (OGT),” Vertical Axis Wind Turbine 200 Mega Watt off Shore Wind Farm (VAWT Off Shore JV)-City of Evanston, Illinois Lake Michigan Project”.

[2]. M. A. Mueller and A. S. McDonald,”A lightweight low speed permanent magnet electrical generator for direct-drive wind turbines”, Institute for Energy Systems, Institute for Energy Systems, Edinburgh University, Edinburgh University, Edinburgh, UK.

[3] Vishal D Dhareppgoaland MaheshwariM Kona-gutti,” REGENEDYNE Maglev Wind Power Gen-eration”, SARC-IRAJ International Conference, 16th June 2013, Pune, India, ISBN: 978-81-92747-8-3.